���������������������������Ŀ

�  ��ͻ  ��ͻ  ��ͻ  ���ͻ  �                 Utility for

�  �     �  �  ��͹    �    �                 antivirus

�  � ��  �  �  �  �    �    �                 researchers

�  ��ͼ  ��ͼ  �����  ���   �

���������������������������������������������������������������������Ŀ

�                                                                     �

�               GOAT File Creator Package, Version 4.2                �

�                   (c) MIG, Igor G. Muttik, 1997                     �

�                                                                     �

�����������������������������������������������������������������������

��������������������������������������Ŀ

� NEW - CREATION OF NE-EXE AND PE-EXE  �

� FILES FOR WINDOWS 3.x AND WINDOWS 95 �

����������������������������������������

�������Ŀ

�PURPOSE�

������������������������������������������������������������������������������

GOAT package is a tool for the antivirus researchers.

The GOAT file generator produces executable victim file(s)

(COM, EXE or SYS), typically called "sacrificial goat file(s)".

These output files are used as baits for the viruses.

The purpose of the programs can be explained using the

following diagram:

����������ͻ      ����������������������Ŀ

� Goats    ������Ĵ You need a goat file �       �����������������Ŀ

� creation �    � � with some special    �������Ĵ use GOAT.COM in �

� task     �    � � internal structure.  �       � "just-one-file" �

����������ͼ    � ������������������������       � mode            �

�                                �������������������

� �������������������������Ŀ

� � You need a series of    �    �����������������Ŀ

�Ĵ goat files of different ����Ĵ use GOAT.COM in �

� � sizes.                  �    � "series" mode   �

� ���������������������������    �������������������

�

� �������������������������Ŀ

� � You need a set of files �    �����������������Ŀ

�Ĵ of the same size with   ����Ĵ use GOATSET.BAT �

� � the different contents. �    �������������������

� ���������������������������

�

� ��������������������������Ŀ

� � You need many identical  �  ���������������������Ŀ

�Ĵ samples to infect them   ��Ĵ use GOAT.COM in     �

� � with the polymorphic     �  � "zillions-of-files" �

� � virus.                   �  � mode                �

� ����������������������������  �����������������������

�

� ��������������������������Ŀ

� � You need many different  �  ���������������������Ŀ

�Ĵ samples to infect them   ��Ĵ use GOAT.COM in     �

� with the polymorphic     �  � "zillions-of-files" �

� virus.                   �  � mode with /U switch �

����������������������������  �����������������������

Using GOAT.COM you can manually select the size, the name

of a sacrificial goat file and vary its internals to meet

the criteria, which the virus uses when deciding "to infect

or not to infect" the victim file. You can enter the size

of a sacrificial goat file in any of given formats: decimal,

hexadecimal or in kilobytes. Size of the victim files can

be as small as 2 bytes and as much as many gigabytes (it is

a 32-bit variable). GOAT.COM is very flexible - it can create

COM, EXE, SYS(COM) and SYS(EXE) files, NE-EXE, PE-EXE with

code at the beginning, in the middle, or at the very end of

the goat file. Files can be filled with zeroes, NOPs, two

types of pattern or even filled with random garbage. You

can add stack segment for the EXE files, vary header size,

and ... many other options are available.

"Series" mode is intended to create a series of bait files

with linearly increasing length. Length increase step is

changeable.

In "zillions-of-files" mode GOAT.COM can create unlimited (you

will just run out of disk space long before) number of files.

Then you can infect this set of files with a polymorphic virus

to test its behavior and properties. This mode is turned on if

'+' (plus) or /Fnnn or /D[nnn] is given in the command line.

GOATSET.BAT produces some sort of "a standard set" of files

of the same size. These files are different (internal contents

or attribute are variable). GOATSET.BAT needs GOAT.COM for the

execution. GOAT.COM should be located in the current directory,

or it should be accessible via PATH environment variable.

The "standard set" of goat files may help you to find out

which files are prefered by the virus (ex.: virus may infect

only COM files starting with JMP). Checking "a standard set"

after virus attack, you can easily understand which files

are infectable.

A small batch file RUN-ALL.BAT will help you to run (i.e., infect,

if you have a resident virus active) all generated bait files.

Text of RUN-ALL.BAT is self-explanatory.

Using GOAT.INI file you can change default behaviour of GOAT.COM

to better meet your requirements.

��������Ŀ

�EXAMPLES�

������������������������������������������������������������������������������

Examine the following examples with comments:

������������������������� "just-one-file" mode ��������������������

GOAT /?                  // gives help screen

GOAT 2 c:\bait.com       // create 2-byte file C:\BAIT.COM

GOAT 100k                // create 100k file GOAT000.EXE

GOAT 10 /b/j             // make short COM file with code at the

bottom, without JMP instruction

GOAT 34 /e2              // create EXE file with 32-byte header

GOAT 8k /e100h           // create long EXE header (100h=256para=4k)

GOAT 200 /e10/9          // create EXE file with a 160-bytes header

// and fill the file with NOPs

GOAT 570 /e/b/j          // make EXE with a standard 512-byte header,

place code at bottom and avoid JMP

GOAT 4000 /iE5h          // make 4000 bytes COM file, fill with E5h

GOAT 8k/r/t62            // make 8k COM file with readonly attribute

and seconds field in timestamp = 62 sec

GOAT 30000 t.exe         // create 30000 bytes file T.EXE, but

with internal COM structure, i.e.,

without EXE header

GOAT 30000 t.exe /e      // create 30000 bytes file T.EXE,

with standard 512 bytes 'MZ'-header

GOAT 2000h /e            // create 8192 bytes file GOAT000.EXE

GOAT 512k /#20h/w        // create 512k NE-EXE file GOAT000.EXE with

20h relocation entries in the EXE header

GOAT 1k /r7              // make 1k Hidden/System/Readonly COM file

GOAT 300k /e40h/o/j/9    // make 300k EXE file, not overlaid, without

JMP, filled with NOPs with 1k EXE header

GOAT 300k /e40h/o/j/9/b  // same, but code at bottom of file

GOAT 300k /p/t18h        // make 300k EXE file and fill it with

00, 01, 02, ... pattern, set seconds

in the file timestamp = 18h = 24

GOAT 1k /n               // make 1k COM file, filled with random bytes

GOAT 1k /u               // make 1k COM file, different from above

GOAT 1k /y               // make 1k SYS device driver

GOAT 1k /y/e             // make 1k SYS-EXE device driver

GOAT 1k /e/k1k           // make 2k EXE file with 1k STACK segment

GOAT 1k /e/k810h         // make 3088 bytes EXE file, code segment

will take 1024 bytes, stack - 2064, and

SP=810h, CS:IP=FFF0:100

GOAT 1k /e /k810h /M512  // make 3088 bytes EXE file, code segment

will take 1024 bytes, stack - 2064, and

SP=810h, CS:IP=0:0

�������������������������� "series" mode ��������������������������

GOAT 1000-65000 1000     // create 65 COM files

GOAT 1000-65000 1000 /E  // create 65 EXE files

GOAT 600-1600 100 /E/L   // create 10 EXE files and assign them

// a previous year's timestamp

GOAT 1k-66k 32k /b/9     // create 3 files. First two will be COM

files with 1k and 33k size, last will

be EXE file. Names assigned: GOAT000.COM,

GOAT001.COM and GOAT002.EXE. Code

located at bottom, filled with NOPs

GOAT 1000h-2000h 200h    // create 9 files: GOAT000.COM (size=

4096) .. GOAT008.COM (size=8192)

GOAT 50k-60k 10h /e2/b/v // create many EXE files with 32 byte EXE-

header, code at bottom. All will have

SS=CS, SP=IP

GOAT 1024k-16384k 1024k  // create 16 files, all EXE. Will

probably result in "Not enough disk

space" error, because files will take

>120MB in total

GOAT 100-1000 100/m50/p2 // make 10 COM files with JMP to the code

located at byte 50, word-pattern filled

GOAT 1k-10k 1k /y/e      // make 10 SYS device drivers, organized as

EXE files with 512-bytes header

�������������������������� "zillions-of-files" mode ���������������

GOAT 100 +               // make 500 100-byte COM files in 10 dirs

GOAT 1k /D               // make 500 1k COM files in the current dir

GOAT 1000 /F400/D5/Q65   // create 2000 COM files of size 1000

in five directories - DIRAAA...DIRAAE

Names are GOATAAA.COM .. GOATCJJ.COM

GOAT 12345h /E2/B/9/D1   // create 500 files. Files will be EXE,

74165 bytes in size, with short EXE

header, NOPs filled, code will be

placed at the end of the files. Names

assigned: GOAT000.COM .. GOAT999.COM

All in one subdirectory - DIR000

GOAT 2000 /F10000/D1000  // create 10 millions of 2000-byte files

in 1000 directories

�������������������������� "test-set" mode ������������������������

GOATSET 1024             // create 52 different files. Twelve

COM files, 34 EXE files and 6 SYS files

(2 nornal SYS and 4 SYS-EXE).

See GOATSET.LOG for the report.

GOATSET 30k              // create 52 files. See GOATSET.LOG

for the report.

�������������������������� "running goats" ������������������������

RUN-ALL                  // run all generated GOAT???.COM and

GOAT???.EXE files

�����������������Ŀ

�USAGE OF PROGRAM �

������������������������������������������������������������������������������

You can see the correct usage of all programs, specifying the

"help" command line switch "/h" (or /?):

GOAT /h

GOATSET /h

Printout of GOAT.COM's help screen is given below:

�����������������������������������������������������������������������������

�

�GOAT  -  Version 4.2  -  (C) MIG, 1997

�

�Usage: GOAT  Size [Filename]  [/switches]               - "just-one-file"

�       GOAT  StartSize EndSize [Increment]  [/switches] - "series mode"

�       GOAT  Size  [+] [/D[nnn]] [/Fnnn]  [/switches]   - "zillions-of-files"

�

�Size - dec, hex or kbytes (Ex.: 1000, 65530, FE00h, 31k, 512K, 2048k)

�If [Filename] is not given creates GOAT000, GOAT001...

�

�Switches:                               /N[nnnn] RND filled (nnnn = seed)

�  /Annnn   set SYS driver Attribute     /P,/P2   use byte or word pattern

�  /B       code at Bottom               /Qnn     use char 'nn' instead of '0'

�  /D[nnnn] Directories (default = 10)   /R[nn]   make file Readonly/Hidden/Sys

�  /E[nnn]  EXE file (nn = header size)  /Tnn     seT seconds = nn (<63, even)

�  /Fnnnn   Files in dir (default = 500) /U       Unique RND fill

�  /Gnn     use 'nn' as NOP replacement  /W[nnnn] Make NE header at offset nnnn

�  /Inn     fIll byte = nn (/9 for NOP)  /X[nn]   replace E9 00 00 with 'nn's

�  /J       no JMP at code start         /Y       create device driver (SYS)

�  /Knnnn   add STACK seg of nnnn size   /Z       use 'ZM' EXE header

�  /L[nnnn] set year = nnnn (/L = oLd)   /#[nnn]  put nnn EXE relocations

�  /Mnnnn   put code at offset 'nnnn'    /!       Make PE executable (Win-95).

�

The square brackets denote optional parameters.

��������Ŀ

�SWITCHES�

������������������������������������������������������������������������������

Short reference of all available switches is given below in

the alphabetical order.

Note that any switches having a parameter could be entered either

in decimal or hexadecimal notation (ex., /i100 or /iE5h).

/Annnn  - set SYS device Attribute (default=0C853h). Have no sense

without /Y switch. (May be placed in the INI file to

change default device driver attribute.)

/B      - place code at bottom of file (default is at start)

/C[n]   - set GOAT.COM selfcheck level (by default equals to 2,

the highest) (/C means /C0, i.e., no checks at all)

/D[nnnn]- create maximum nnnn subdirectories (default=10)

/D (or /D0) means make goat files in current directory.

(this switch turns on "zillions-of-files" mode if it was

given from the command line)

/E[nn]  - create EXE file (if size > 65280 - done automatically)

If nn is given file will have an EXE header of

nn paragraphs long (just /E will produce standard

512-bytes EXE header). Header size is limited to

about 32k.

/Fnnnn  - create maximum nnnn files in a subdirectory (default=500)

(this switch turns on "zillions-of-files" mode if it was

given from the command line)

/Gnn    - use nn byte instead of NOP (when, say, /J/B are used)

/Gnn should be used with *caution* - bad selection of nn

will produce goats which hang when executed. This switch

is useful when included in the INI file. See also /X.

/H, /?  - Help screen

/Inn    - use fill byte nn instead of standard zero-fill

(ex., /i100 or /iE5h)

/J      - remove JMP at code start (by default it is present)

/Knnnn  - add nnnn bytes of STACK segment to the bottom of executable

file (stack segment is filled with 'STACK' string by default)

Can be placed in the INI file. Parameter /K given in the

command line can supress /Knnnn definition given in the

INI file.

/L[nnnn]- assigns year 'nnnn' to a created goat file(s). If given

without number (just /L) - assigns previous year (say,

if current year is 1996 goat will get year 1995).

Please note that /L switch affects both the timestamp

of the file and the timestamp embedded in the PE-header.

/Mnnnn  - place code in the middle of the file exactly at offset

nnnn (nnnn can be in the range 0..filesize, even when

the file is >64k).

/N[nnnn]- fill goat file with pseudorandom bytes. The parameter

(if given) is a random number generator seed.

RNG uses multiplicative congruental method with

2**32 period.

/O      - do not make long EXE (>256K) with internal overlay structure

This option affects only EXE files longer than 256k.

By default such EXE goats get number of pages in the EXE

header like in a 256k EXE file (i.e. the rest of the file

is not loaded in memory when the file is executed).

/P[n]   - fill free file space with pattern. Switch /P or /P1 means

byte pattern, while /P2 - word pattern. Byte pattern looks

like is - 00, 01, .. FE, FF, 00, ..  Word pattern is -

0000, 0001, ...FFFF, 0000. Both types of patterns

are "anchored" to the location (i.e., byte pattern at,

say, offset 2BCE will be CE CF D0 D1...; word pattern at

the same location will be CE 2B D0 2B D2 2B...).

/Qnn    - use char 'nn' (ex., 65='A') instead of '0' when naming goat

files (ex., /Q65 will do: GOATAAA.COM, GOATAAB.COM,...)

/R[nn]  - setting nn in range 1..7 you can produce any combination

of Readonly/Hidden/System attributes (/R or /R1 sets

Readonly bit, /R2 - Hidden, /R4 - System).

/S      - turn off selfchecking of the created goat file. Also

makes goat code a bit shorter (about 80 bytes instead

of 286). See also switch /7.

/Tnn    - set timestamp seconds field = nn. DOS stores the seconds

of the file creation in the rounded to the closest even

value. So odd values are not allowed. Seconds field

should be in the range 0..62 and even (ex., 0, 1Eh, 62,...)

/U      - guarantees that contents of each generated file is unique

(fills file with random bytes and and do not reset random

generator before creation of each file).

However, if you make a set of files (in "series mode" or

"zillions-of-files mode") many times - you will get an

identical set every time (as the same seed produces the

same pseudorandom sequence). In "just-one-file" mode PC

clock is used to give a seed for pseudorandom generator

and therefore you will get unreproducible garbage

filling for each goat.

/V      - set SS:SP equal to CS:IP

/W[nnnn]- make NE-EXE file. By default NE-EXE header will be

located at offset 400h. The offset value is rounded

to the closest 32-bit boundary. NE-EXE header should

be in the first 64k of the EXE file, so offset limits

are [40..FF8C] (but it shouldn't also overlap with the

code or with relocations, if any). See also /2 switch.

/X[nn]  - kills E9 00 00 and replaces it with given bytes.

Switch /X without parameter is useful only when the

code is not at the goat file start (/B or /Mnnnn were

used). Otherwise it has the same effect as just /J.

/Xnn should be used with *caution* - bad selection of nn

might produce goats which hang when executed.

If there are values patched in the body of the program

where [90 90 90] was - they are used in the first place.

/Y      - create device driver (SYS file). May be combined with

/E to make SYS-EXE files.

/Z      - make 'ZM' EXE header instead of 'MZ'.

/2      - merely changes target OS in the NE-EXE header to OS/2

(default is Windows). Can be specified twice to get

"unknown" target OS.

For PE executables increases OS MAJOR, USER MAJOR

and SUBSYS MAJOR fields (defaults are, 1, 0 and 3,

respectively). Can be placed in the INI file.

/9      - fill free file space with NOPs (default - with zeroes)

(it is just a shortcut for /i90h).

/7      - enables beeping when selfcheck fails (works both for

selfcheck of GOAT.COM generator and for produced goat

files which have selfchecking code).

/#[nnnn]- specifies the number of dummy relocation entries in

the EXE header. Relocation make happy a couple

of viruses, which refuse to infect goats with no

relocations. Relocations are dummy - they do not

affect the goat code. If the code of the goat is

at the file start (just after EXE header), then

relocations will affect bytes just after the goat

code (after B4 4C CD 21). If code is at file end (/B),

or in the middle (/Mnnnn), then relocations are made

to the bytes just after file end (or to the stack

area if it's present). Switch /# without parameter

can suppress /#nnn definition given in the INI file.

For normal EXE files relocations start at 1Ch, for

Windows NE/PE executables - at offset 40h.

/!      - Make PE-executable file. By default PE-header will

be at offset 100h. Any generated PE executable has

exactly 5 sections (CODE, .icode, DATA, .idata and

.reloc). Option /! also sets size of the EXE header

to 40h bytes (use /Ennnn to change if you wish).

See also /2 switch.

The switches in the command line (like /E) give you the control

over the victim creation. You can place the code at the beginning

of the goat file or at the bottom. You can select the filling

byte (00, 090h or even pattern), the presence of JMP at the code

start. You can force creation of EXE file. You can even change

size of EXE header and control values loaded in SS and SP. You

can prohibit the creation of overlaid EXE files, etc. Most of

the command line switches can be applied simultaneously.

Value, specified in the switch /Mnnnn (nnnn is 32-bit value

and you may use "k" and "h" modifiers), has the following

limitations:

for EXE: (header size) < nnnn < size-286 (no /S)

for EXE: (header size) < nnnn < size-78  (with /S)

for COM              3 < nnnn < size-2

If some switches are conflicting (say, "GOAT 1000 /P0/P2", where

/P0 - byte pattern and /P2 - word pattern), - the last is used

(same applies to the switches in the INI file).

Some combinations of switches are not allowed. For example, you

cannot set /J, /B and /P for any COM file simultaneously (if

code is at bottom and no JMP is present - pattern will appear

as a code at CS:100).

Switch /Knnnn adds stack segment at the bottom of the file

(always EXE). Size of the stack segment is limited:

16 < nnnn < 65536

Please avoid small and odd values in /K, because when the goat

file created with such an odd stack is run it could easily hang

a computer or cause "Exception #13" (QEMM frequent warning),

when SP goes through the stack segment boundary (i.e., half

of word is written at SS:0000 and other half - at SS:FFFF).

Note, that the size specified in /Knnnn is added to the main

size of the file (i.e., GOAT 1k /k1k will produce 2048 bytes

EXE file). If the main size of the file does not end on the

paragraph boundary (i.e., main size is not divisible with 16) -

stack segment will start on the first paragraph after the end

of the main file size.

Switches /Fnnn and /Dnnn given in the command line automatically

turns the program into a "zillions-of-files" mode (or use '+').

You can specify the desired number of files and subdirectories

to create. By default, 10 subdirectories with 500 files in each

are created. Note, that if any directory already exists (say,

DIR004), it will be simply skipped and no new files will be

created inside.

If you specify just /D switch this will allow creation of

zillions-of-files in the current directory (same as /D0).

��������Ŀ

�INI FILE�

������������������������������������������������������������������������������

You may like to put your preferences (signature, switches,

filename templates, etc.) into a separate file - GOAT.INI

Use any text editor to create or modify INI file.

Note that not all switches are suitable for placing in the

INI file as some of them are "direct-action". They immediately

force certain mode of operation: say, if you place /E in the

INI file you will no longer be able to create goats of COM

type. Here is the list of switches suitable for INI file

(although, the last five are not very useful in the INI):

# 2 7 9 A C D F G I K L N O P Q S X  and  J R T U V

The following are not suitable for the INI file (not

mentioning /? or /H switches):

! B E M W Y Z

The sample INI file is given below:

����������������������������������������������������������������������

�;

�; sample INI file

�;

�Motto="Antivirus test file."       ;all output bait files will carry

�                                   ;this string.

�files=BAIT             ;files will be BAIT000.COM, BAIT001.COM, ...

�dirs=HEAP              ;directories created - HEAP000, HEAP001, HEAP002

�STACKfill="*MYSTACK"   ;fill stack with '*MYSTACK*MYSTACK*MYSTACK'

�SYSnames="DRIVERXX"    ;this string is inserted into SYS header.

�Switches=/F800/D15     ;make 15 dirs, 800 files in each. 12000 in total.

�Switches=/L            ;always put previous year in a goat timestamp

�Switches=/iF6h         ;always fill free file space with 0F6h byte

�Switches=/g40h         ;use INC AX instead of NOP

�Switches=/q65          ;use GOATAAA..GOATJJJ instead of GOAT000..GOAT999

�Switches=/O            ;never make overlaid EXE files

�Switches=/K200h        ;if EXE - always add 512 bytes stack at eof

�

GOAT.INI may be located in the current directory or in the path

of started GOAT.COM. The first location has priority over the

second. GOAT.INI may not exist. In that case program uses

built-in defaults.

The following statements are allowed in the INI file:

Motto=             ;defines signature for all created goat files

;(not present on default)

files=             ;filename template for GOAT.COM (default=GOAT)

;it can be empty (ex., files=)

dirs=              ;directories template for zillions-of-file mode

;it can be empty (ex., dirs=)

STACKfill=         ;stack fill string (default=STACK)

SYSname=           ;driver name (default=GOATXXXX)

Switches=          ;any switches you like (multiple lines allowed)

Any comments may be added after a semicolon.

Filename template (files=) and dir template (dir=) can be

empty (this will make all letters of the name variable:

000-999 or whatever was defined by /Qnn).

Motto string, defined in the INI file appears in the created

goat COM file immediately after JMP (if any) or after the goat

code (if no JMP). In the EXE file it appears after the goat

code, if code is at the start. If space at the beginning is

free (ex., /B or /Mnnnn switch moved goat code away from the

beginning) - motto will appear at the EXE file start (after

the EXE header). Motto string is limited to 64 symbols. To

include spaces in the motto string use quotes. Note, that

/J/B and /J/Mnnnn switch combinations supress motto in the

goat file.

Switches /F and /D given in the INI file just modify the

built-in default values for the number of files and directories,

created in "zillions-of-file" mode. This does not turn on

this mode. To do so you will need to specify '+' (plus)

in a command line explicitely.

Filename and subdirectory templates are limited to 5 symbols,

because programs always add '000' and then start incrementing

this number until it becomes '999' (or 'AAA' ...'JJJ' if you,

say, redefined the naming with /Q65 switch). Dot is not

allowed in the template. Any string exceeding the limit

of 5 symbols will give the error message:

"Error in the INI file line #xxx"

The processing will be aborted. The same error will appear if

you give any wrong switch (say, /@) or parameter value (say,

/T68 or /i500).

Stack fill string is limited to 32 symbols (you can use

even unprintable characters, like ASCII 01, etc.).

The strings may be quoted. To enter a quote itself - duplicate

it inside the quoted string - ex. "File is ""readonly"""

will be converted to a string: 'File is "readonly"'.

Driver name, defined using "SYSname=", is limited to 8 symbols.

This name is right-padded with 'X' symbols, if name has less

than eight symbols.

Note that any INI file switch settings is overridden by

command line switches.

����������������Ŀ

�SIZE LIMITATIONS�

������������������������������������������������������������������������������

By default GOAT.COM produces sacrificial file of COM type.

This applies to any given size, which meet the following

criterion:

2 < Size_of_COM < 65280

The magic number 65280 is a maximum size of COM file, which

should fit in a segment size (64k) taking off PSP size (256):

65536 - 256 = 65280.

When placing the code at the bottom of the COM file having

size about 64K, code may lay too close to SS:SP (for COM

files: SS=CS, SP=FFFE) and the program may hang your system

when run, because stack might overwrite the code.

Therefore, if the spacing between IP and SP is less than

64 bytes, the goat generation is aborted and the goat file

is not created (You will see a warning - "Goat IP will be

too close to SP. Abort!"). If you absolutely need such a

file - use switch /Mnnnn to place code at the very bottom

of your file. With /M switch the test is not performed,

because if you specify some value in /M switch you probably

understand what you are doing!

When the size specified in the command line is greater than

65280 (or equal to), EXE file is generated automatically (you

do not need to write /E switch explicitly). Such a file will

have an EXE header in the beginning. Because of the presence

of EXE header, the size limitations are the following:

513 < Size_of_EXE < free_disk_space (with /E switch)

33 < Size_of_EXE < free_disk_space (with /E2 switch)

When you need to create EXE file shorter than 65280 bytes,

use /E (or /Z) command line switch explicitely.

When creating a series of bait files in a "series" mode,

the size can exceed 65280 bytes limit at some point. All

files longer than 65280 will be automatically turned to

default EXE files (i.e., with a standard 512 bytes 'MZ'

header) and file extension will change to .EXE.

���������������������������������Ŀ

�BAIT FILES INTERNALS AND PRINTOUT�

������������������������������������������������������������������������������

The sacrificial goat file itself contains a small program,

which displays its type (COM, EXE or SYS), size in hexadecimal

and in decimal (only when goat file is of enough size, i.e.,

space for code itself is at least 74 bytes).

Sacrificial goat file consists of the two parts: the small

portion of code (of fixed size, always being 2 bytes or 74

bytes) and a block of zeroes, NOPs or pattern of variable

size (00..FF, 0000...FFFE or random pattern). Zeroes (NOPs

or pattern) take all space of the file, free from the code.

EXE files have additionally an EXE-header. Non-used part of

the EXE header is always filled with zeroes. SYS files have

SYS header, strategy and interrupt routines in addition to

the code given below.

Two types of code can be placed into the victim file (COM/EXE).

They are given in the table:

����������������������������������������������������������������Ŀ

� # � Code                               � Size, dec � Size, hex �

����������������������������������������������������������������Ĵ

� 1 � int 20h                            �      2    �     2     �

����������������������������������������������������������������Ĵ

� 2 � This code just prints message      �    ~80    �   ~50h    �

�   � with the file type/switches/size   �           �           �

����������������������������������������������������������������Ĵ

� 3 � This code apart from printing      �   ~256    �  ~100h    �

�   � a message selfchecks the size      �           �           �

�   � of host goat file and reports      �           �           �

�   � the size changes (if any)          �           �           �

������������������������������������������������������������������

Dots (/....) in the message printed by the goat file are replaced

with 4 last command line switch letters. Any switches specified

in the INI file are not included. All command line switches

modify this field (namely - ABGIJKLMNOPRTUVWXZ!#279). Switches

that change the mode of GOAT.COM operation (namely - CDEFQY)

are not reflected. Switches /E,/Y are not reflected because

file extension (EXE) or printout (SYS) is itself an obvious

indication.

The output of a sample goat file (the size of the sample was

100 bytes and command-line switch /B was given) will be

the following:

"Goat file (COM/b..). Size=00000064h/0000000100d bytes."

File type (COM/EXE/SYS), real numbers and switch letters

(if any) are inserted into the goat file message at the

moment of creation.

�����������Ŀ

�BAIT NAMING�

������������������������������������������������������������������������������

Usually GOAT.COM generator creates output sacrificial files in

the following order: GOAT000.COM, GOAT001.COM, GOAT002.COM, etc.

The same applies to EXE files, and the order is ascending:

GOAT000.EXE, GOAT001.EXE, GOAT002.EXE, etc. If some file in

a row (say GOAT050.COM or GOAT050.EXE) already exist - the

next file number is selected automatically (it will be

GOAT051.COM or GOAT051.EXE). This does not apply to SYS

files (i.e., GOAT000.COM and GOAT000.SYS are allowed).

This naming strategy is used to give some freedom to

companion viruses.

Switch /Qnn is used to make it more difficult to virus-

writers to build viruses which avoid infecting the goat

files using the presense of digits in the filename.

If, say, you specify /Q65 in a command line (or in the

INI file) all generated goats will have 'AAA', 'AAB', 'AAC'...

instead of '000', '001', '002'... You can use any char

if its ASCII code is in the range [33; 126], but note that

some characters are not allowed by DOS (like ":\><|/.").

Such characters will be simply skipped.

GOATSET.BAT produces 52 files of the same size. There will

be 12 COM, 34 EXE, 2 SYS and 4 SYS-EXE files. GOATSET.LOG

is created after each start of GOATSET.BAT. Inspect this

log file (GOATSET.LOG) file in order to get the report about

the internal structure of generated files, the error messages,

etc.

Note, that definitions, given in the INI file may change

default file (and sudbirectories) naming.

����������Ŀ

�EXE HEADER�

������������������������������������������������������������������������������

By default, EXE header is 512 bytes in size. You can change

EXE header size using /E switch with parameter. For example,

/E2 produces shortest possible 32-byte EXE header. Size of

EXE header is limited to about 32k.

By default, EXE header starts with 'MZ' signature. Use /Z

switch to make 'ZM'.

When the sacrificial file is less than 256K bytes, the

first two fields (pages and remainder in the last page)

are assigned according to the real file size. But when

code is placed at the beginning of bait file and when

the limit of 256K is exceeded, the number of pages is set

to 2 (EXE header size plus code size) and code on the last

page is set to 64 bytes. This technique makes an "overlaid"

EXE file. It is absolutely needed to enable execution of

files bigger than the amount of free conventional memory

(say, for execution of a 700K file or a 10MB file).

Switch /O does not affect the internals of EXE files

smaller than 256K.

If you specify /B switch (or /Mnnnn or /Knnnn switches),

the overlay creation mode is automatically turned off

(same as if you write /O in the command line). Otherwise,

the code (or stack) will be unaccessible and such a file

will not run (DOS will give error message, when attempting

to start such a file).

If you need a goat file with, say SS:SP equal to 400:1234,

use one of the following calls:

GOAT 4200h /K1234h

GOAT 4020h /K1234h /E2

Value of SP=1234h is obvious. To calculate file size take

needed SS, multiply it on 10h (to convert paragraphs into

bytes) and add 200h (bytes in standard header) or 20h (for

short EXE header).

��������������Ŀ

�DEVICE DRIVERS�

������������������������������������������������������������������������������

To create device driver (SYS) file use switch /Y. Device driver

file can be in two formats - simple driver (a'la COM file) and

EXE-file driver (with EXE header, a'la SMARTDRV.EXE or SETVER.EXE).

Files of the first format can only be invoked from CONFIG.SYS and

they will print message "Goat file (SYS). Size=..." when DOS

requests an initialization of the driver. Second format (SYS&EXE)

can be placed in the CONFIG.SYS and can also be invoked from the

DOS command line as a normal EXE file. In both cases this driver

file prints the same message.

Minimal size of the device driver is around 150 bytes (including

SYS header). For EXE drivers this limit increases (it should

include additionally the size of the EXE header).

Note that device drivers, being simultaneously an EXE files, has

a "(SYS)" type designator inside, but are always named as .EXE

files (to enable execution from the command line as a normal

executable file). Such files can exceed 64K only if device

driver routine is placed in the first segment of the goat file

(because pointers to Strategy and Interrupt routines in the

driver header are limited to 16-bits). So, a command

"GOAT 70k /y/b" is illegal, but "GOAT 70k /y/m63k" is OK.

Default device driver attribute is set to 0C853h (most usable

bits are set). You can vary attribyte with /Annnn switch (or

even place this witch setting into the INI file to override

default setting).

����������Ŀ

�ERRORLEVEL�

������������������������������������������������������������������������������

GOAT.COM returns ERRORLEVEL=0 after the successful operation.

GOAT.COM returns ERRORLEVEL=1, when  output victim file was

not created or if the current operation failed (say, no more

space when creating zillions-of-files). The error message

will be printed.

GOAT.COM returns ERRORLEVEL=2 when user pressed Ctrl-Break

(this feature is used in batch file GOATSET.BAT).

GOAT.COM returns ERRORLEVEL=3 when selfchecking has failed

(Note, that /C switch turns off selfchecking).

�������������������������������Ŀ

�SELFCHECKING OF GENERATED GOATS�

������������������������������������������������������������������������������

Generated goat files (except SYS files) check their own size

by default (this option works only under DOS 3.0 or later).

If goat file has been changed - the difference is reported

both in decimal and hexadecimal notation:

Goat file (COM/....). Size=00002710h/0000010000d bytes.

Size change=0CDAh/03290d.

If size change is more than 64k - the goat file will print

question marks instead of digits:

Goat file (EXE/....). Size=00002710h/0000010000d bytes.

Size change=????h/?????d.

If you wish size change report might be accompanied by a PC

speaker's BEEP. Use switch /7 to activate this option.

Note that not all infections will be reported - file system

infectors (like DIR-II, Byway) will not be detected. Cavity

viruses (like BootEXE, Zerohunt) and companion viruses (like

Gold Bug, Little Brother, HLLC) will be missed too. Most

of the others are caught without problems (please let me

know if you find any).

To suppress the selfchecking code use switch /S. For example

this may be needed to create shorter goats, as selfchecking

code is long enough.

Goat file returns ERRORLEVEL=0 if selfcheck was OK and

non-zero if selfcheck failed.

������������������������Ŀ

�SELFCHECKING OF GOAT.COM�

������������������������������������������������������������������������������

GOAT.COM program checks its own health. Selfchecking has three

levels (selectable with /Cn switch):

0 - no checks

1 - check of the file disk image

(includes file time stamp and COM extension)

2 - check of the file disk image and

of the file size (default level)

If the checksum of the file on disk is invalid you will

see a warning after program termination:

"GOAT.COM  altered!

Size change=00001000h/000004096d bytes."

If you need to turn off file contents checking - use switch

/C0 (or simply /C).

Therefore, do not try to pack GOAT.COM with PKLITE or

similar compressors. That will result in the "GOAT.COM

altered!" report.

If program file extension is not COM you will see (if switch

/C is not specified):

"GOAT.COM  altered by companion virus!"

All selfchecking is automatically turned off (same as /C0) for

DOS versions prior to 3.00 (because the location of the original

file on disk cannot be found).

�����������������Ŀ

�PATCHING GOAT.COM�

������������������������������������������������������������������������������

This paragraph is for experienced users only! Do not try

to modify GOAT.COM if you do not 100% understand what is

written below.

Although the core of the GOAT.COM program is protected from

modification (partial checksumming of GOAT.COM file is done),

you can patch the program to better meet your requirements

because data area near the end of the file is not checksummed.

When doing any of descibed modifications to GOAT.COM preserve

the original timestamp of the file - it is checked too. (For

example, you can use HIEW to modify the file without touching

the timestamp.)

Near the eof you can see the code of the created GOAT files

(starting with bytes E9 00 00 E8 ...), its messages ("Goat

file (COM)" and "bytes." You see also image of the EXE header

(4D 5A ...), SYS file code (56 E8 00 00 ...) and NOPS (90 90 90)

replacing the jump (E9 00 00) when you specify /J switch.

You can patch these areas to make GOAT more unique. That's

worth doing because some viruses may start trying to

recognize the goats produced by this package and will

refuse to infect them. For example, you can set your own

MINALLOC, MAXALLOC, CHKSUM fields in the EXE header.

You might like to change 90 90 90 to any reasonable and

appropriate processor commands (like ex., MOV/INC/ROR/etc.)

But still you will have to fit into 3 bytes only!

You can also modify "Goat file " string to whatever you

want. This will change the message your goats are

printing when executed. And again - the size of this

string should be preserved.

��������������Ŀ

�AUTHENTICATION�

������������������������������������������������������������������������������

If you have PGP program installed on your machine you can

check the authenticity of GOAT.COM file.

I include below my PGP public key. It can be used to check

the validity of GOAT.COM (even for future versions).

-----BEGIN PGP PUBLIC KEY BLOCK-----

Version: 2.6.3i

mQCNAiz3uywAAAEEAKRvzeHdrUT1tlgIbpPoVmI51qiDctKIKGlY82cLCsnKs3Mh

c0d73bjprJqpgFGy/DmLARJj15FcnTjwT2lS4DCBX/+Um3PMZTUpoDDcHwV+LbC+

CfUf/4vupkjdhqIt8LzFwoPYaPH0wG6Wh3WJpaqYkRsgPQMagzqaFa593YhRAAUR

tCNJZ29yIEcuIE11dHRpayA8bWlnQGx0LnBoeXMubXN1LnN1PokAlQMFEDL/TBE6

mhWufd2IUQEBRIUD/0hZPyvKyKn7kmyJ2k9JPyAOAzjMjM6UHGpLx3A1XaF2R0ia

amkYfm1xCHYsp82wmBhyROImpE7DurU1k/KmxuMPS8UifxsE3NkwUnc1NRH7VOvc

lH/Pa9nfQ/lItMQNEcr9DQOYmCl3uI+taXW4Wog43spFyCNNLWti3aHgMGBetCNJ

Z29yIEcuIE11dHRpayA8TUlHQGx0LnBoeXMubXN1LnN1PokAlQMFEDLmEDdQK/tU

ud7FuQEB6AYD+waV+Ji92OWh22mnZdoBsdPXEbkRCh2J0+KTCqtFtZ6GdNBSxaFi

4wpPgqkxlMIwDtRZLU/dl+NSKzhk45EDU5f6blM5OnhC32b1BlnFY99jzfsM4xAT

aVDWnCn+zhnshIcU8elTgz1p/y3GJhGyxCw9lEAa26XBWtSSdmS/SpygiQCVAwUQ

MW0uxDqaFa593YhRAQE/HgP/dc+nrly5FKbpEF5vLN/5WvKt2JXGoY2Y3/UdZBCT

HQr5BKMGhnvwnArdagi+fgdHhzNgsrxp1n5FT5piyzwsKBSQOAw5YCgpNqpwD5XN

/XiMjXgEkSJiST1didLrelHw8xmE96VjWDFvtdy/q0QAjjkLNLNWxoyvg+tYIkxa

Gg2JAJUCBRAs+z1cMIAgxAYM3PEBAQbGA/9sIuNXpUhhk0j+oSc5s8+xHBVwxa9B

9Atu926CoxavNrzXjQMvSS0pKcszuH2ZkLto0FGs5X7+8DGtfMi01K/yHRxHlsFJ

vrChfv/Qm8ADCUCkl12Fz5uR2Nd11LcR+iqmuS5U5kByxGnUsPAKErVdqSPpPVAW

cGJiTsrONyTwNokAlQIFEC7bAyY2VpfGMt2Y2QEBjTAEAJGSlkFtC2zHN4IDKQKb

g+4aFiOmBfclfTv64PgXPmkTL8IAZ10HZYKm75H6+zZ+/9j5ZgLQeUz+GEq+qJXH

MigNIdHbIRwi4ny9SFMi+oyzA78GMqJtUkUAg+RXziQC4xxf9GE+8PXq7b17dsb8

5WzlX9HC4IOyYJ3cN5XnVsIFiQCVAgUQLXMOD5qVRRb01TllAQGZJwQAw0qxT4HC

iGF+oYdXnwGuFj5xHCvMgk+V199LMn7kNOpIbmdOK6vLaa/pDDHkyMnaPlTOSlsG

QwWqJjodnIXFs7NglYfRlN9zfKQcV9zoMxhWBmjpdkL2CJsq13Rh0ttLnHVSQZCW

timEMBfN6xAFQIWFZJDSE+fgkwTK9Sk4XHW0HjxJZ29yLk11dHRpa0B1ay5kcnNv

bG9tb24uY29tPokAlQMFEDL/SUI6mhWufd2IUQEBWEwD/3w60OlWajPMgqaKEue8

tGB5W4jBanWamRxQvTGJDT8+e1FSOiC+flIPyHRtJxDBvnbFqjG7Ls4H7bE8NjXk

fO2nL2anYefQyN+hXm5ninbWq2lRQJjJQrUn03EszwyBIfmqMMLejxfVHIiIMdcG

76datnxIhFdEOuTrDQ0dCBKTtBU8bWlnQG1pZy5waHlzLm1zdS5zdT6JAJUDBRAy

5g9mOpoVrn3diFEBAa8NA/9keCujOMshcp9JoO73X6OWBaajHoTCZfdC14SaBSkH

22zafIWtEZSF2mCoKt3TGdNHoZnNZOjQ1yfPdm9bUO4pfw43+XT5mxyo0RDnMa8d

rkc4pMJ2qvorz+HR/Ed4lo5E+RhoUXfVazivLPTPUaCDzSKttEkenZ+7AWcZqOhT

MbQVPGlnb3JAZGlhbC5waXBleC5jb20+iQCVAwUQMuYOozqaFa593YhRAQHYwwP8

DeRVwKVaNPcgULB8Wx3Ag1gnLSV5sm6Jm59pck9+5Unbe+tFgCzeWSBK+qzHjFau

yAgVvmkqOWf/7ioEnLW7XGhDA1bI0mYYkNLvXlKlXCDqIVVegyOZsjw3B6Q8IhUp

1SxwZlC9CvgodEiDZwnHC+h2GgSRnYkfHt4wJIzHBUk=

=bfzJ

-----END PGP PUBLIC KEY BLOCK-----

Provided PGP key has some signatures of well-known antivirus

people but if none is recognized - feel free to contact me to

check the validity of this public key). You can also try

"finger mig@mig.phys.msu.su" to get my PGP key. First, add

my public key to your PGP keyring:

PGP -KA GOAT.DOC

To check the validity of GOAT.COM use the following command:

PGP GOAT.ASC GOAT.COM

������������������������Ŀ

�VERSIONS ON THE INTERNET�

������������������������������������������������������������������������������

I would be happy to send the latest version of GOAT package

to everybody who needs it. Do not hesitate to contact me

personally via Email. Copies of the program are available

on Simtel, Garbo and their mirrors worldwide. You can

usually download the latest copy from:

ftp://ftp.informatik.uni-hamburg.de/pub/soft/virus/progs/tools/goat42.zip

�������Ŀ

�HISTORY�

������������������������������������������������������������������������������

Version 4.2 -  A bug with /Qnn switch was fixed. The bug

demonstrated itself only when more than 10

files were created in one subdirectory.

Version 4.1 -  DOC file was slightly rewritten - false alarm of

McAfee SCAN was found ("STRAT.486" - see below

"BUGS" section).

Version 4.0 -  All programs were joined into one - GOAT.COM.

Goat file output changed - now it includes the

printout of switches used when created this file.

All limitations on the number of files (used to

be 1000) were removed (now, after 000...999 go

AAA...ZZZ, etc.). New switches introduced - /Qnn,

/Gnn, /L, /!, /#, /7. Redefined switches - /W, /X,

/S. Parameter added to switches /Enn, /Rnn.

Goat code is now able to selfcheck the goat file.

GOAT.COM now checks its own size/checksum and

reports the change (if any).

Version 3.1 -  Some small improvements were made: programs

now do not report "Your PC might be infected"

under Novell. New switch /U introduced. Switch

/D behaviour was improved. And, finally, EXE header

now contains 1C as an offset of relocation entries

list (although the list itself is empty; helps to

handle a couple of viruses, which need non-zero

word in 001C).

No bugs were reported for more than a year since

release of 3.0 so no fixes were needed.

Version 3.0 -  Goat file contents changed - the string,

indicating type of goat file, added to the

printed message (COM/EXE/SYS).

Device driver creation mode added: switches

/Y (make SYS file) and /Annnn (set device

attribute) appeared. "SYSname=" definition

added to the INI file processor (this name

appears in the SYS file header).

Help screen redesigned - now switches are

listed in the alphabetical order.

FLOCK.COM, when running on an almost full drive

entered very long loop - corrected.

GOATSET.BAT creates now 6 device driver files.

Version 2.1 -  Motto string added to the INI file.

INI file processor allows quoted strings now

in the "Motto=" and "STACKfill=" definitions.

New switches /N[nnnn] (random fill with

possible seed definition) and /X (supress

Motto string defined in the INI file) added.

GOAT.INI was not found by FLOCK and GOATS

thru path, fixed.

Version 2.01 - Some portion of code (common to GOAT, GOATS

and FLOCK, version >1.5) alerted F-Prot (it

reported "Probably new variant of BootEXE").

I rewrote it in order to avoid this false alarm.

Version 2.0 - INI file processing added.

FLOCK.COM appeared instead of FLOCK.BAT.

FLOCK.COM switches /F (# of files in dir)

and /D (max # of subdirectories) appeared.

Selfchecking level added (switch /Cn).

Switches /O and /V do not force EXE now

(to allow including /O, /V into INI file).

Word pattern (/W) fixup corrected.

GOATSET operation (when GOAT.COM selfcheck

fails) was corrected.

Version 1.6 - Switches /K, /M, /W, /C, /I added.

Many enhancements in the code were made.

Selfchecking is double now (in-memory copy

and file on disk), check for companion virus

infection added. Rare conflict of /R and /T

switches fixed. Removing of non-fully created

readonly files fixed.

Version 1.5 - Switch /P (pattern fill) added.

Selfchecking of timestamp added.

/J/B/E or /J/B/S forced NOP-filling

for EXE files, corrected.

When making a series of files with GOATS.COM

on 5 (and 67) byte boundary, additional JMP

appeared even with /J switch, corrected.

Version 1.4 - File numeration changed, it is common

for EXE and COM files now. Ex., if GOAT000.COM

exist, created EXE file will be GOAT001.EXE

(to give some freedom to companion viruses).

Version 1.3 - Added switch /Tnn. Bug with attribute

in GOATS fixed (always set 0000, even

with /R switch). When modified, GOATS.COM

reported "GOAT.COM probably infected!",

corrected. Minalloc field in EXE header

changed to zero (ver. <1.3 always set 20h).

Version 1.2 - DOC file written. BAT files FLOCK and

GOATSET added. Decimal printout of goat

file size added. Added rotating symbol

"|" (when creating goat files longer

than 64k). Selfchecking added.

Version 1.1 - Many switches added (/S,/O,/B,/J,/Z,/V,/9,/R).

Version 1.0 - First version, without documentation,

supported only switch /E. No decimal

size in the printout of goat files.

����Ŀ

�BUGS�

������������������������������������������������������������������������������

All EXE files >=1024k, created with /O and /E2 switches

(short header, not overlaid) hang old DOS versions (<5.0)

upon execution. For a very similar file with normal 512-

bytes header, DOS behaves normally. It reports "Program

too big to fit in memory" or "Cannot execute GOAT000.EXE".

This DOS bug is no longer present in DOS 5.0 and 6.0.

Old DOS versions were obviously confused with such a strange

goat file (the file without overlaid structure and greater

than 640K cannot fit into DOS memory and have no real sense).

DOS should report a load error, because a load size is

definitely >640K. Unfortunately it hangs instead.

One known bug: if EXE file generated is longer than 1MB and

code is at bottom (ex., 1025k /e/b) - CS:IP will be assigned

incorrectly. But I do not feel like correcting this as DOS

cannot load/execute such files anyway.

Some versions of McAfee SCAN (at least 2.5.2 dated September

1996) produce a false alarm ("Found the STRAT.486 virus") in

all SYS files created using GOAT package and in the GOAT.COM

generator itself (versions 3.x and 4.x). The alarm is caused by

the code responsible for creation of the SYS files. This has

been reported to McAfee and fixed in November 1996 release

2.5.2 of McAfee SCAN.

���������������Ŀ

�ACNOWLEDGEMENTS�

������������������������������������������������������������������������������

I am greatly acknowledged to Frans Veldman (Esass, the Netherlands).

Discussions of some aspects of a goat file generation were very

fruitful. I also got some useful suggestions from Igor Danilov

(Sald, St.Petersburg) and Vesselin Bontchev (FSI, Iceland).

I benefited from the usage of HIEW.EXE (Hacker's View)

program by Eugene Suslikov. HIEW.EXE, being the viewer of

an executable files, is very useful for the examination of

the contents of goat files. I recommend HIEW for all users of

GOAT package. Try the following Internet URLs to get a copy:

http://palantiri.spb.su/English/Misc/GSS/toolsoft/hiew.html

http://www.kiarchive.ru:8098/msdos/sys/hack_tools/

ftp://ftp.cdrom.com/.2/dresden/edit/hiew524.zip

������Ŀ

�FLAMES�

������������������������������������������������������������������������������

Send flames, questions, etc. directly to the author

(preferably via Email).

All ideas, comments and corrections will be appreciated.

Sorry for possible errors in this manual - English

is not my native language.

������Ŀ

�AUTHOR�

������������������������������������������������������������������������������

Dr. Igor G. Muttik

Virus Laboratory

Dr Solomon's Software Ltd.

Alton House, Gatehouse Way

Aylesbury, Bucks, HP19 3XU

United Kingdom

Email addresses : Igor.Muttik@uk.drsolomon.com

MIG@uk.drsolomon.com

igor@dial.pipex.com

MIG@mig.phys.msu.su

MIG@lt.phys.msu.su

MIG@sands.co.uk

Phone: +44 1296 318700 ext.2756

Fax:   +44 1296 318734