Documentation of QCONFIG

The philosophy of QCONFIG is to try and determine most aspects of

machine hardware in the system.  The angle of attack is through

BIOS, Dos Int 21h, Device Drivers, and even straight from known

memory locations.  Whatever gets the job done is done.

We have tried our best to make the code machine independent.  It

should not matter which DOS version (later than 3.30 and QCONFIG

runs in an OS/2 VDM) is being run.  It also should not matter if

you are on a 8088 or a 80486.

Options

-------

-?  or ?    Displays options (help)

-A          Displays all supported Microchannel adapters

(can be used with the -O option to redirect to file)

-C          Displays additional information on Async communications

ports

-D          Displays more detailed QCONFIG information

includes : logical drives, adapter configuration for

MC, model, submodel, revision, bios date,

fixed disk sectors/heads/cylinders

-E          Displays current environment

-I          Displays CONFIG.SYS and AUTOEXEC.BAT

-M          Displays all recognized machines by Model, SubModel and

Revision

(can be used with the -O option to redirect to file)

-O          Redirect output to QCONFIG.OUT

-Ofilename  Redirect output to filename

For example:

QCONFIG -Ops2data.fil

would redirect the output of QCONFIG to ps2data.fil

-P          Pause the output when the screen is full

Note, this also writes a file (QCONFIG.OUT) in the

default drive/directory

-Q          Does not display redirect message

key=value   Define key with value for machine info output

There is a limit of 10 keys at present

For example:

QCONFIG User="Norris Couch" "Location Site"=Boca

Output would include:

User            : Norris Couch

Location Site   : Boca

Notes:

------

PC/3270s and AT/3270s are no longer supported.

The DOS version must be 3.30 or later.

The calculated machine speed is incorrect when run in an OS/2 VDM

and will not be displayed under OS/2 unless you have the DOS

setting for hardware timer set on for the session running

QCONFIG.

The memory simm detection method in not supported when run in an

OS/2 VDM and therefore memory simms will not be displayed under

OS/2.  This is an OS/2 limitation.

The planar id may be incorrect (unknown) on some Model 80s when

run in an OS/2 VDM.  This is an OS/2 limitation.

The maximum about of real memory that will be reported is 64MB.

Problems and Suggestions

------------------------

Please append to OS2EWS CFORUM on OS2BBS or send a message to my CIS

ID [Norris Couch, 73763,2656] to report any problems.

Thanks:

------

This program would not reached this level of reporting without the

generous help of Sam Detweiler, Jeff Knauth, Cindy Merkin and

Akihiko Togami.