I am about 90% complete writing BBL a public domain 32 bit

Forth-83 compiler. (I still have to write FORGET, WORDS, a

relocator, a post-fix assembler and finish the editor). If you

an experienced Forth/Assembler hacker and are willing to test,

debug, criticize and possibly even write code for BBL in its

current form, please drop me a line.  It is written in Microsoft

Assembler MASM version 4.0.

BBL will be officially released in October along with a suite of

other public domain products: Abundance, a Forth Based database

language; DOIT, an unattended file transfer system that uses the

international X.25 net with X.3, X.28 async access (written in

C); and HDTEST, a program to find and map out marginally bad

tracks on hard disk whether or not they are in use (also written

in C).  Source code will be available for all products.  There

will be an article in the October Byte on BBL/Abundance.

BBL uses hashed (for fast compilation) dictionaries with

separate disposable headers (so the nfas can be thrown away

after compilation).

BBL uses 32 bit stack elements, but has 8, 16, 24, 32 and 64 bit

operators as well.  Its NEXT inner interpreter is quick: LODS

JMP AX,  (not JMP [AX]).   Colon definitions are compiled to 16

bit tokens -- the offsets of Cfas in the CS: segment.  (all cfas

live in the first 64K). The sum total of assembler code is

normally limited to 64K, but high level colon definitions can

span a megabyte.

For speed, words like @ and CMOVE expect absolute Seg:offset

addresses.  The code was optimized for speed rather than

compactness.  It makes great use of the 8086 string handling

instructions.

It keeps the top element of the data stack (not a pointer -- the

actual value) in CX:BX.  DI is nearly always kept 0.  DS:SI in

the FORTH IP. SS:SP is the FORTH SP and SS:BP is the forth RP.

The down-side is there are 5 different kinds of HERE.  There is

no >NAME and >BODY is somewhat slow.  Because the code uses

absolute addresses, if overlays are loaded in a different

location in RAM it needs some weird processing before it will

work.  To have relocatable code, you need to compile twice for

different base addresses and compare the two memory images to

create the relocation bit map.

send a disk mailer to:

Roedy Green

Canadian Mind Products

#11 - 3856 Sunset Street

Burnaby BC CANADA

V5G 1T3

(604) 435-2957