KANODE.DOC

Notes on using KA-Nodes in FWD.MB for forwarding connects:

----------------------------------------------------------

(For use with W0RLI V9.xx and later software and Kantronics KPC-2, KPC-4 and

KAM with V2.85 ROMs or later.)

Date: March 25, 1989

From: Greg Day - KC8JN @ KC8JN

Wintersville, OH 43952

GENERAL INFORMATION

-------------------

This information is provided for those SYSOPs using Kantronics KA-Nodes in

their forwarding connect paths to remote BBSes.  The W0RLI software is designed

to handle NET/ROM type connects automatically, but because the KA-Node issues a

sign-on message when accessed, has different status messages, and the W0RLI

software is not designed to handle this type of information, it is necessary to

implement a special sequence to handle these KA-Node connects.

ALTERATIONS TO FWD.MB

---------------------

The following is the standard sequence to be placed in the FWD.MB file within

a path list:

xC kanode-n

Sm destcall-n

R!

R!

R!

R?###LINK MADE

Where......

x = C or N depending on whether or not this is s direct connect (C) or a

connect made from a NET/ROM node (N) - see the FWD.DOC documentation

with the W0RLI software

kanode-n = call sign of the KA-Node with which you wish to be connected

(-n is the SSID, if any)

m = mode for KA-Node, C is a direct connect, X is a cross (Gateway)

connect (to the opposite port on the KPC-4 or KAM - if the GATEWAY

parameter is ON)

destcall-n = call sign of the next station with which you wish to be

connected (-n is the SSID, if any)

Examples of connect path records from FWD.MB:

CC K8AL-1          (make initial connect to NET/ROM node)

NC NG8P-1          (connect to the KA-Node from the NET/ROM node)

SC AD8I            (make a direct connect to the destination BBS)

R!

R!

R!

R?###LINK MADE

FA0023AD8I

@C:\BBS\AD8I.FWD

*** EOF

OR......

CC K8AL-2          (make initial connect to NET/ROM node)

NC NG8P-1          (connect to the KA-Node from the NET/ROM node)

SX K1LT-2          (cross connect using other radio port via GATEWAY)

R!

R!

R!

R?###LINK MADE

NC AD8I            (continue with NET/ROM node style connects)

FA0023AD8I

@C:\BBS\AD8I.FWD

*** EOF

Be sure to type in the required information, especially the "R?###LINK MADE"

line, EXACTLY as shown, otherwise the forwarding will fail!

SPECIAL CASES

-------------

Please note that you should test ALL KA-Nodes you use to be sure of the data

they send upon connect.  You may need to make the following changes for those

KA-Nodes that have earlier ROMs (prior to V2.85) or have the NTEXT message set

to null:

Earlier KA-Nodes or those with V2.85 ROMs and NTEXT set to NONE......

Delete one (1) line of "R!" to allow for only two message lines being

sent upon connect.

Earlier KA-Nodes with a different prompt character - V2.85 uses a "?" but

earlier ROMs have a ">" instead......

Change the "?" (after the "R" in the LINK MADE line) to a ">" - this

is the prompt for some earlier KA-Nodes.

CONCLUSION

----------

If you have any questions regarding the use of the Kantronics KA-Nodes in

forwarding with the W0RLI Mailbox, pleae contact me via packet "@ KC8JN",

located in eastern Ohio.

73, Greg - KC8JN