STANDARD MICROSYSTEMS CORPORATION

Sun PCNFS v3.5

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the SUN PCNFS version 3.5 NDIS

driver files from the SuperDisk to your PC.

o  How to prepare your PC's operating system so you can attach

to the network.

Note: This driver supports all 8216 (Ultra), 80xx (Elite), and

80xx (PLUS) Series ISA and Micro Channel Ethernet adapters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy

all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

PREPARING YOUR OPERATING SYSTEM

1. Insert the Installation and Connections diskette into a

disk drive (A:\ or B:\).

2. Type: INSTALL

3. Press the Enter key.

4. Specify the destination drive letter (usually C) and press

Enter.

5. Specify "Ethernet" and press Enter.

6. Select the "NDIS" option.

7. Follow the screen prompts to copy the PCNFS files to your

destination drive.

8. When prompted, press Enter to configure PCNFS.

9. Specify the PC name, IP address, server name, and server IP

address. If you need assistance, refer to the "Configuring PC-NFS"

section of your Sun PC-NFS Installation Guide.

10. Follow the screen prompts to complete the installation.

11. When the installation is complete, select Quit to exit the

configuration program, defer the boot, and return to DOS.

12. Using any text editor, add the following line below the

NFS_NDIS.SYS driver specification in your CONFIG.SYS file:

device=c:\lanman\SMC8000.dos

Note: The 'c:' shown in the line above indicates the destination

drive you selected during the PCNFS installation.

13. Use any text editor to edit the PROTOCOL.NFS file in the

\LANMAN subdirectory in your Destination Directory (do not edit

the \LANMAN subdirectory on your SuperDisk):

- Replace:

[YOUR-MAC-MODULE]

with:

[SMC]

- Replace:

drivername = yourmac$

with:

drivername = SMC8000$

- Remove the line:

option1 = value1 etc.

- Replace:

bindings = YOUR-MAC-MODULE

with:

bindings = SMC

- Locate the file PROTOCOL.NFS in the \LANMAN subdirectory on

your destination drive and rename it to PROTOCOL.INI. Refer to

your DOS manual if you need assistance.

- Add the following NETBIND statement just before the NFS RUN

statement in your AUTOEXEC.BAT file:

C:\LANMAN\NETBIND

NFSRUN

Note: The 'c:' shown in the line above indicates the destination

drive you selected during the PCNFS installation.

14. Turn your computer off and then on.

SMC and Standard Microsystems are registered trademarks; and EtherCard

Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite 16C Ultra,

Elite Series, EliteView, EZStart, SuperDisk, and UltraChip are

trademarks of Standard Microsystems Corporation.  Other products and

company names are registered trademarks or trademarks of their

respective holders.