��������     ���� ��������

���  ���   ����   ���  ���

�������  ���� ��� ��������

������� �������� ��������

���  ��� �������� ���  ���

��������      ��� ��������

A Full Screen Simulator for 8048 Compatible Microprocessors

William Luitje

11 February 1995

�����������������������������������������������������������������Ŀ

���� Background ���������������������������������������������������

�������������������������������������������������������������������

I  wrote this simulator as part of my work on a consulting job  a

few years ago.   There is still quite a lot of work one could  do

to improve it,  such as simulating the timer,  providing a way to

manually  change  RAM and ports,  simulating changes  in  inputs,

command files...  A LOT!  Still, I think it's useful, as is,  for

learning the 8048  instruction set and debugging small  programs.

I  hope  it's  useful  to you;  if you use it,   please  see  the

registration section at the end of this document (relax, no money

is involved).

With  the  work I mentioned above and some features  specifically

for  an educational environment,  this program would make a great

tool   for  teaching  microprocessor  programming.     Commercial

programs  like  this  cost from $200  to $300.    If  I  make  an

educational  version  of  S48,   I   envision  a  site  licensing

arrangement  which  would cost a very modest amount per  student.

If  you are an instructor or administrator and are interested  in

pursuing this, contact me at the address given at the end of this

document even if you use a different micro in your course.

�����������������������������������������������������������������Ŀ

���� Introduction �������������������������������������������������

�������������������������������������������������������������������

S48   simulates the instruction set and some of the hardware  for

the 8048,  8049,  8050, 8021, 8022  microprocessors originated by

Intel and now widely second-sourced.  The simulator handles these

hardware parameters:

* 4096 bytes of ROM

* 128 bytes of internal RAM

* 256 bytes of external RAM

The  hardware  registers  and  flags  and  RAM  are  continuously

displayed  on  screen  and  are updated as  each  instruction  is

executed.  There is a disassembly window showing the instructions

as they are executed.   You can trace through instructions one at

a time or execute them continuously until a breakpoint is reached

or you stop the program manually.

The  hardware requirements to run S48  are quite simple;   you'll

need  a  PC with a CGA,  EGA,  or VGA compatible video card  (the

program does direct screen writes so you'll see snow using  CGA).

A  color  display is not strictly necessary,   but  is  certainly

helpful in reading the screen easily.  Obviously, the faster your

CPU,   the  faster the simulation will run.  A  4.77  MHz  PC  is

adequate for small programs.   The software requirements are even

simpler: you'll need some version of DOS, S48 and an 8048 program

to simulate.

�����������������������������������������������������������������Ŀ

���� Sample Program �����������������������������������������������

�������������������������������������������������������������������

I've included a very simple sample program for you to play  with,

called demo.bin.  demo.asm is the source file and implements a 16

bit  counter in software.   There is an interrupt service routine

which  captures  the low byte of that counter and  stores  it  in

consecutive  locations  in  external RAM when  the  interrupt  is

invoked.    It's short enough that I can reproduce  its  assembly

listing here:

.org   0

0000 00                   nop

0001 04 06                jmp    start

0003                      .org   3

0003 00                   nop

0004 04 14                jmp    intsvc

;

0006 27          start    clr    a

0007 B8 18                mov    r0,#24

0009 A0                   mov    @r0,a

000A A8                   mov    r0,a

000B A9                   mov    r1,a

000C 05                   en     i

000D 18          loop     inc    r0

000E F8                   mov    a,r0

000F 96 0D                jnz    loop

0011 19                   inc    r1

0012 04 0D                jmp    loop

;

0014 F8          intsvc   mov    a,r0

0015 D5                   sel    rb1

0016 90                   movx   @r0,a

0017 18                   inc    r0

0018 C5                   sel    rb0

0019 93                   retr

001A                      .end

You'll also need an understanding of the 8048 instruction set and

architecture.  You won't find much help in this document.

�����������������������������������������������������������������Ŀ

���� Tutorial �����������������������������������������������������

�������������������������������������������������������������������

The 8048 program to be executed must be in binary form,  like you

would use to program an EPROM,  not in "hex" form.   To start the

simulation, just give the name of the binary file as an argument,

e.g:

s48 demo.bin

Take  a  minute to familiarize yourself with the  layout  of  the

screen:

����������������������������������������������������������������������������Ŀ

�Cmd:                                                                        �

� 26 bytes read from demo.bin                                                �

���PC� �AĿ�PSW�CY�AC�F0�F1�RB�STK��TE�TF�IE�                                �

��000� �  ���08�� �� ��0��0��0���0���0��0��0�                                �

� �MB� �Bus��P1�P2�P3�P4�P5�P6�P7� �T0�T1�II�                                �

� ��0� ��  ��  �  �  �  �  �  �  � �� �� �� �                                �

�    ��00��10��20��30��40��50��60��70��ͻ          �000�00����NOP�������     �

� 00 � ��                               � 08        001 04 06 JMP  006       �

� 01 � ��                               � 09        003 00    NOP            �

� 02 � ��                               � 0A        004 04 14 JMP  014       �

� 03 � ��                               � 0B        006 27    CLR  A         �

� 04 � ��                               � 0C        007 B8 18 MOV  R0,#18    �

� 05 � ��                               � 0D        �����Breakpoints���Ŀ    �

� 06 � ��                               � 0E        ��� ��� ��� ��� ����    �

� 07 � ��                               �������������������������������ͻ    �

�    ��00��10��20��30��40��50��60��70��80��90��A0��B0��C0��D0��E0��F0��͹    �

� 00 �                                                                  �  08�

� 01 �                                                                  �  09�

� 02 �                                                                  �  0A�

� 03 �                                                                  �  0B�

� 04 �                                                                  �  0C�

� 05 �                                                                  �  0D�

� 06 �                                                                  �  0E�

� 07 �                                                                  �  0F�

�    ����0F��1F��2F��3F��4F��5F��6F��7F��8F��9F��AF��BF��CF��DF��EF��FFͼ    �

������������������������������������������������������������������������������

The  top  line is where you type your commands to the  simulator.

The  second  line is where any messages from S48  are  displayed.

The hardware registers and flags are displayed with a light  blue

background  at  the  top  left  of  the  screen.    There  is  an

instruction disassembly window at the top right with the  current

instruction (the one to be executed next) shown in reverse video.

Below   it   are  the  5  instructions  following   the   current

instruction.    Above  it will be displayed the 6  most  recently

executed instructions.  Below the disassembly window is where the

breakpoints are displayed.

The large L shaped box shows all of RAM.   The top part shows 128

bytes of internal RAM and the lower shows 256  bytes of  external

RAM.   There isn't enough room on the screen to have blank  space

between  the columns so they are distinguished by the  background

color.   The first column contains locations 00  through 07;  the

second holds 08 through 0F, and so on.  The light blue background

in internal RAM shows the active register bank.

When  the simulator is started the processor is in  reset.    The

values  of registers which are initialized by RESET,  e.g.    the

Program Counter,  are shown;  registers which are undefined after

RESET, such as the Accumulator and RAM are shown as blank.

Let's do some simulation.  Give a Trace command by pressing the t

key  and  return.   The NOP instruction has now moved  above  the

current  instruction  line,  the PC has changed to 001   and  the

current instruction is JMP 006.   Give another trace command  and

the  current  instruction  changes to CLR A.    Notice  that  the

instructions  after the current instruction have now  changed  to

reflect  the jump.   Their color has changed,  too,  to  make  it

easier to see that that has happened.  (This is especially useful

for following conditional jumps.)  Trace again and notice how the

Accumulator is initialized to zero.

Now  let's  watch the simulation run for a while.   Give  the  Go

command  by  pressing the g key and return.   As  the  simulation

runs,  notice how memory location 0 counts up quickly and  memory

location 1 counts slowly.  Press any key to stop the simulation.

To invoke the interrupt, press the i key and return.   Notice how

the  current instruction changes to the first instruction of  the

interrupt  service routine at location 003.   Trace  through  the

rest  of  the  interrupt  service  routine  and  notice  how  the

background  of the register banks changes as the active  register

bank is switched.

To  demonstrate setting a breakpoint,  type "b 11"   and  return.

This  will  set a breakpoint at the instruction which  increments

the high byte of the 16 bit counter. Now give a go command and in

a few seconds the simulation will stop at location 11.  To remove

the  breakpoint,   use the kill command with the address  of  the

breakpoint you want to remove.   In this case, "k 11"  and return

will do the trick.

To Quit the simulator, press q and return.

�����������������������������������������������������������������Ŀ

���� Command Summary ����������������������������������������������

�������������������������������������������������������������������

You've  seen all of the commands except for Reset and Port in the

tutorial;   here  we'll  summarize them and give  a  little  more

information about them.   Only the first letter of the command is

parsed  and  it can be either upper or lower case.   Some of  the

commands can take a numeric argument which is always expressed in

hexadecimal and need not be given.

g #   Go: Start simulating at address #.  Simulation stops when a

breakpoint  is reached or when a key is pressed.   If #  is

not given,  simulation starts at the location given by  the

Program Counter.

t #   Trace:  Trace  #  instructions.   If # is not  given,   one

instruction is traced.

b #   Breakpoint:  Set  a breakpoint at the given address.    The

command has no effect if a number is not given.

k #   Kill  breakpoint:   Remove  the  breakpoint  at  the  given

address.    The  command has no effect if a number  is  not

given or if there is no breakpoint at that address.

p # # Port value:  Set the port  specified by the first # to  the

value  specified by the second #.   Two parameters must  be

given.

r     Reset:  Simulate a  reset.   The Program Counter is set  to

zero and other registers are set as appropriate.  Registers

which are not affected by RESET are not changed.

i     Interrupt:  Simulate an interrupt.   The Program Counter is

set  to  the  location of the  interrupt  service  routine,

0003H, and further interrupts are disabled.

q     Quit:  Quit the simulation.  Come back again soon!

�����������������������������������������������������������������Ŀ

���� Registration �������������������������������������������������

�������������������������������������������������������������������

While this software may be freely copied and used,  I  do request

that  you  register  it if you use it.   Just send  me  either  a

postcard   or   some  e-mail  telling  which  program   you   are

registering.   I'd prefer a local,  picture postcard;  after all,

whether  it's great or whether it's a pit,  the place  where  you

live is probably interesting in some way, right?

This  is only partially an ego massage for the author.   I  enjoy

writing  useful  utilities  and  have  several  other  pieces  of

freeware  out floating around the net.   I'd like feedback  about

which of them is worth expanding and supporting.  Postware really

works!    I  support the programs which are most used.   I'm  not

planning on adding features to this program, but I might if I get

enough postcards.

�����������������������������������������������������������������Ŀ

���� Conditions of Use ��������������������������������������������

�������������������������������������������������������������������

The program has not been tested against a real live 8048  because

I  don't  have access to an 8048   hardware  development  system.

That's why I wrote this simulator in the first place!   The Intel

data  books  I used were ambiguous in places so  I  made  guesses

about  how  some of the subtler points of the  simulation  should

work.  (E.g.,  does INC set AC?  I  guessed not.)   There may  be

instances  where  I  guessed wrong.   Bug  reports  are  welcome,

especially  when  you  can provide documentation  about  how  the

simulation really should work.

So, keep in mind that this software is provided as is.  I make no

guarantees about it except that it will take up space on any disk

you place it on.  Further, any software you write with the aid of

this simulator is your responsibility.   Verify that it  operates

properly  with  actual  hardware before you use it  in  the  real

world.

William Luitje

2677 Wayside Drive

Ann Arbor, MI 48103

USA

Internet:  luitje@m-net.arbornet.org

�����������������������������������������������������������������Ŀ

���� History ������������������������������������������������������

�������������������������������������������������������������������

Version 1.0  1 Aug 94

Initial public release

Version 1.1 11 Feb 95

Added command to set ports

Added color change of prospective disasssembly window after

jump, branch or interrupt.