============================================================================

Punter Protocol Version 1.04

(C)Copyright 1989

Data Technology Corp.

All Rights Reserved.

============================================================================

This is a basic documentation to help the unregistered user get the

protocol installed in his or her system with greater ease.  However, more

detailed documentation, and a more advanced protocol is included on the

registered disk.  You will also be entitled to low-cost updates of

the documentation and the protocol itself.

============================================================================

Punter Protocol 1.04          Table of Contents                           1

============================================================================

I. Basic Overview .............................................. 2

Shareware Contract ........................................ 2

System Requirements ....................................... 2

II. Installation ................................................ 3

III. Setup ....................................................... 4

Execution ................................................. 4

Options ................................................... 4

Uploading and Downloading ................................. 6

ERRORLEVELs ............................................... 7

IV. Registration ................................................ 8

How to Register ........................................... 8

Registration Form ......................................... 9

Update Request Form ....................................... 10

CREDITS ......................................................... 11

============================================================================

Punter Protocol 1.04        I. Basic Overview                              2

============================================================================

This new protocol standard for the IBM has been implemented to allow

greater compatibility of IBM and Commodore machines.  Since Punter has been

a standard on the Commodore for years, it has become customary for Commodore

owners to use this reliable protocol.  This protocol offers a bridge between

Commodore and IBM machines.

1. Shareware Contract

---------------------

Please feel free to implement this external protocol in your bulletin

board system or on your favorite terminal, but please note that this

software is by no means free.  It is the result of dedicated time and hard

work.  Please support the programmers in their endeavors to bring you low-

cost, quality software.  As you will note, the registration fee is quite

reasonable.  This program is a part of Data Technology Corporation's new

shareware or user-supported program.  It is designed to offer new software

to the consumer on a trial basis.  If you do not feel that the software

meets your needs within a 30-day period, you obviously will not be required

to register, however, if you do use this product, please register and

support the programmers that have brought you this quality software.  Also,

you are encouraged to distribute the shareware product as you wish as long

as no part of the program or documentation has been altered.

2. System Requirements

----------------------

This program has been designed to run on the IBM Personal Computer and

compatible PC clones.  This program is only guaranteed to run under

Microsoft DOS or IBM Personal Computer DOS on an IBM PC, XT, AT, or 100%

Compatible Clone, although it may run on a variety of systems.

Basic System Requirements:

- IBM PC/XT/AT (Or Compatible Clone)

- 64K RAM

- Serial/Asynchronous Port (COM1 - COM4)

- Modem/Null Modem

- 1 Double Density Disk Drive or Better

The system described above will allow you to run the protocol, but you will

most probably need a communications package or bulletin board system to

fully utilize the protocol.  Memory requirements will increase well above

64K when a communications program is installed.  A Modem is required if you

wish to transfer data with dial-up systems.  However, if you only wish to

transfer data between computers, a null modem (basically an interconnecting

cable between serial ports) will be required.  You will also need some sort

of recording media for the program and files to be sent or received.  A

single-sided, double-density disk is the basic requirement.

============================================================================

Punter Protocol 1.04          II. Installation                             3

============================================================================

Installation of this protocol is pretty straightforward.  You will be

required to do the following steps:

- Copy the program "PUNTER.EXE" (Included With This Documentation) to

the desired diskette or directory where you will execute it.  If you

want, you may make a special directory for this purpose.  If you do

not know how to do this, please consult you DOS documentation.

- If you wish, you may also have a directory for uploads, and a

directory for downloads.  If you have a diskette based system, you

must either use the same diskette with the program on it, or use

another diskette if you have more than one drive.  It will be

explained how you can do this for either system.

- You may also be required to set up batch files for your specific

system.  It will be explained later how to create batch files for

your needs.

============================================================================

Punter Protocol 1.04             III. Setup                                4

============================================================================

The following information is designed to help you get the protocol

working on your system.  It will center on specific requirements and

needs of the user.  Examples will also be provided so that you may see how

to install the program.

1. Execution

------------

To execute the protocol, you will need to look at your system and

requirements.  On many systems, you will be required to make a batch file so

a bulletin board system or communications package will be able to utilize

the protocol correctly.  Examples for Procomm Plus and Telix will be

provided since they are the most prominent packages.  If you wish to install

it on a bulletin board system, please consult the documentation for

installation of external protocols.

2. Options

----------

In order to get a list of the command line parameters that the protocol

will accept, type the name "PUNTER" at the prompt with no command line

parameters.  Make sure that the program "PUNTER.EXE" is on the current disk

and in the current directory.

You type:

Punter <return>

It responds:

Punter* Protocol Ver. 1.04

(C)Copyright 1989

Data Technology Corporation

All Rights Reserved.

* MS-DOS Implementation of Steve

Punter's Original "C1" Protocol.

USAGE:

PUNTER [S] [R] [Z<BlockSize>] [P<PortNum>] [B<BaudRate>] FileName

Command Line Parameters:

S - Send Punter

R - Receive Punter

P - Define Port Number (1-4)

B - Define Baud Rate (300-115200)

Z - Maximum Block Size (40-255)

The last command line parameter will always be the filename.

Transfer Aborted.

Exit Code: 1

This shows you the possible combinations in which you may use the protocol.

Each command line parameter must be explained.

The "S" or "SEND" command line parameter tells the protocol that it will be

sending a file rather than receiving one.  If you are sending a file, this

============================================================================

Punter Protocol 1.04             III. Setup                                5

============================================================================

is a required command line parameter.  To use this parameter, you will be

required to execute the file like this:

PUNTER S <Other Parameters Will Follow>

This will initiate the protocol and put it in send mode.

The "R" or "RECEIVE" command line parameter tells the protocol that it will

be receiving a file rather than sending one.  If you are receiving a file,

this is a required command line parameter.  To use this parameter you will

be required to execute the file line this:

PUNTER R <Other Parameters Will Follow>

This will initiate the protocol and put it in receive mode.

The "P" or "DEFINE PORT NUMBER" parameter defines the communications port

that the protocol will attempt to use.  The current implementation of this

protocol supports communications ports one through four. (COM1 - COM4)

Please consult the documentation for your serial port if you are not sure

the number of your communications port.  On many serial ports you will be

required to SET the communications port number with DIP switches or jumpers.

If you are installing you serial port for the first time, it is suggested

that you use COM1 or COM2 since they are the most compatible settings.  To

set up the protocol to send a file using COM2 or port number two, you must

execute the file like this:

PUNTER S P2 <Other Parameters Will Follow>

If you are planning to use COM1, you will not be required to include this

command line parameter.  The protocol will assume that you are using COM1.

The "B" or "DEFINE BAUD RATE" parameter specifies the baud rate at which you

want the protocol to run.  You may have baud rates from 300 to 115200 baud.

Your serial port and modem must both be able to support the specified baud

rate.  To set the protocol to receive a file at 1200 baud, execute the

program like this:

PUNTER R B1200 <Other Parameters Will Follow>

If you wish, you may not include this parameter.  If you do not include the

baud rate parameter, the protocol will use the baud rate that the serial

port was previously initialized to.  If the port has not already been

initialized, it is suggested that you use this parameter.

The "Z" or "DEFINE MAX BLOCK SIZE" parameter tells the protocol the maximum

block size to use.  This is the number of bytes that the protocol will send

in each "packet" of data.  If line conditions are good, you will most likely

use a large block size since the transfer will be much faster.  However, if

line conditions are bad, a small block size should be used to increase

reliability.  For most purposes, a large block size will do!  Allowed block

sizes are 40 to 255 bytes per block.  This parameter only has meaning when

you are sending a file.  If you are receiving a file, the SENDER will decide

the proper block size.  To send a file with 100 bytes per block, do the

following:

============================================================================

Punter Protocol 1.04             III. Setup                                6

============================================================================

PUNTER S Z100 <Other Parameters Will Follow>

For most purposes you will use a block size of 255 since it is the fastest.

If you wish to use a block size of 255, you will not be required to include

this parameter since it defaults to a block size of 255 bytes per block.

The "FILENAME" parameter defines the file that is to be received or sent.

This parameter is MANDATORY.  If you fail to include a file to be sent or

received, the transfer will be aborted.  You may also specify a drive and

directory with the filename if the file is not in the current directory or

on the current drive.  To send a file called "FILE.ARC" on drive D in the

directory "\UPLOADS," do the following:

PUNTER S D:\UPLOADS\FILE.ARC

As you can see, the drive letter comes first, the directory comes second,

and the actual filename comes last.  To receive a file called "THISFILE.ARC"

and place it in the current directory, do the following:

PUNTER R THISFILE.ARC

This will place the file in the current directory on the current drive.  If

you wish to have the file placed on a different drive and retain the drive's

current directory, only specify the drive like this:

PUNTER R D:THISFILE.ARC

This will put the file "THISFILE" on drive D in its current directory.

3. Uploading and Downloading

----------------------------

If you are running under a system that uploads and downloads, you will

be required (in many instances) to create a batch file to process special

data that is sent by the main program.  You will need a separate file for

uploading and downloading.  Procomm Plus (a very popular communications

package) would use the following batch file for downloading:

PUNTER R P2 %1

The "%1" is the filename that will be provided by the communications

package.  As you can see, the port is set to use COM2.  The batch file for

uploading would look like this:

PUNTER S P2 %1

The "%1" again is the filename that will be provided by the communications

package.  If you wished to use a port besides COM2, you would simply specify

a different port number.

On Telix, the program also will send the port and baud rate.  Although, as

mentioned earlier, the baud rate will default to the last baud rate used, it

is sometimes good to specify the baud rate.  The following batch file would

handle downloading on Telix 3.11:

PUNTER R P%2 B%1 %3

The %2 would be the port as sent by the package, the %1 would be the baud

============================================================================

Punter Protocol 1.04             III. Setup                                7

============================================================================

rate, and the %3 would be the specified filename.  For uploading, you would

use the following batch file:

PUNTER S P%2 B%1 %3

Again, the %1, %2, and %3 would be the specified parameters as sent by the

terminal.

A test upload in direct mode or with your batch file should yield the

following if you send the file D:\MODEM\PUNTER.ARC:

Punter Send Session Beginning.

Drive: D:

Path: \MODEM

File: PUNTER.ARC

Communications Port: 1

Baud Rate: 1200

Block Size: 255

If a carrier has been established, the following will also be displayed:

File Size: 21561

Est. Time: 5 Min(s)

Certain parameters will vary under certain conditions.

3. ERRORLEVELs

--------------

Depending on the result of the send or receive session, DOS's

ERRORLEVEL parameter will be set.  If the transfer is successful, then the

ERRORLEVEL will be set to 0.  If it is unsuccessful, it will be set to 1.

These are the standard settings for most systems that utilize ERRORLEVELs to

find out if the transfer was successful or not.

============================================================================

Punter Protocol 1.04          IV. Registration                             8

============================================================================

1. How to Register

------------------

If you use this product, you are encouraged to register.  Registration

offers many benefits.  Initially, you will receive a file containing

explicit documentation on this product.  Secondly, you will be entitled to

low-cost updates.  The updates will also contain new documentation.  To

register, please print this document, fill out the REGISTRATION form, and

mail it to the address given below.  There is also an UPDATE form included

to show you how updates are requested.

============================================================================

Punter Protocol 1.04          IV. Registration                             9

============================================================================

2. Registration Form

--------------------

Product          : IBM Punter Protocol

Form Date        : 01/24/89

System Type      : IBM PC or Compatible Clone

Software Media   : 1 Diskette

Hardware Req.    : Base System

64K RAM

1 Double Sided/Double Density Disk Drive

Serial Port (COM1-COM4)

Modem

Registration Fee : $15.00

Mail Product To:

_________________________________

Name

_________________________________

Address

_________________________________

City           State          Zip

Please enclose fee in check or money order as stated above,

and fill out your name and address in full.  You will receive

the product stated above 7 - 14 days after your form has been

received.  Thank you.

Mail To:

DATA TECHNOLOGY CORPORATION

1383 Collins Rd.

Colorado Springs, CO 80920

============================================================================

Punter Protocol 1.04          IV. Registration                            10

============================================================================

3. Update Request Form

----------------------

Product          : IBM Punter Protocol (Update)

Form Date        : 01/24/89

System Type      : IBM PC or Compatible Clone

Software Media   : 1 Diskette

Hardware Req.    : Base System

64K RAM

1 Double Sided/Double Density Disk Drive

Serial Port (COM1-COM4)

Modem

Update Fee       : $5.00

Mail Update To:

_________________________________

Name

_________________________________

Address

_________________________________

City           State          Zip

Please enclose fee in check or money order as stated above,

and fill out your name and address in full.  You will receive

the update as stated above 7 - 14 days after your form has

been received.  Thank you.

Mail To:

DATA TECHNOLOGY CORPORATION

1383 Collins Rd.

Colorado Springs, CO 80920

NOTE: To receive this update, you MUST be on record as a

registered user.  If the address is not that of a registered

user, personal checks will not be cashed and cash will be

sent to the return address.  Thank you.

============================================================================

Punter Protocol 1.04                 Credits                              11

============================================================================

o IBM is a registered trademark of International Business Machines

Corporation.

o Punter is an MS-DOS implementation of Steve Punter's original "C1"

protocol.

o Procomm Plus is a registered trademark of Datastorm Technology, Inc.