Nice Capture v1.2a for Novell Netware 3.x.

By K.F.Soft, Denmark, Copyright May 1994.

Usage of this program is free. The program may not be sold. It can be

distributed in the original ZIP-file at no charge.

Nice Capture is distributed as is, without any express or implied warranty.

Portions of NCapt Copyright (C) 1983-1990 Novell Inc. All rights reserved.

If you decide to use this program please send me a little note with the

following information:

Type of organization.

Number of servers.

Number of users.

Number of workstations.

Thanks in advance!

Comments, bugs and suggestions to kf@mmf.ruc.dk

Purpose

Nice Capture provides a nice full-screen interface to the capture function

in Netware 3.11. It acts as a front end to capture.exe and endcap.exe.

There is a possibility to gice long desribing names to the queues, and to

set the standard redirection parameters for each queue.

Certain queues can be disabled.

No config file is needed, as the setup's are stored in the Netware bindery.

Screen saver activates after 5 minutes.

Supports mouse control.

Syntax

NICECAPT

Interface

When NiceCapt is started it searches all attached file servers for

available queues, and displays them in alphabetical order.

If a queue has been assigned a long name, this name will be displayed.

Otherwise the queue name will be displayed. Queues with equal names on

different servers are only displayed once, since they probably refer to

the same printer. Two entries are allways present. They are

"End Capture" and "Capture To File".

At the bottom of the screen the current capture status of the three ports

will be displayed.

Use the up and down arrow keys, <PgUp>, <PgDn>, <Home> and <End> to scroll

in the list, or use the mouse to select a queue. Use the left and right

arrow keys or the mouse to select the desired port (lpt1, lpt2, lpt3).

Press <Enter> or double-click the mouse to select a queue. After

capture.exe has been run, the result is displayed in a box. If

capture.exe couldn't be run, an error will be displayed.

If you choose "End Capture" the selected port will be set to local mode.

If you choose "Capture to file" you will be prompted for a file name.

The default file name is "captfile".

Help will be available with <F1> in a later version.

Installation

Place nicecapt.exe in SYS:PUBLIC or in another user-accesible directory

in all users' standard path. Also be sure that capture.exe and endcap.exe

is in the standard path.

For each queue you should define a long describing name and standard

capture parameters. This is done with the included program setprop.exe

(see description below).

If you want to hide a queue (such as the mailqueue) to the user of

NiceCapt, you can disable the individual queues with setprop.exe.

This will be described below.

SETPROP.EXE

The general syntax is

SETPROP /x queuename propertyname ["value"]

To define a long name for a queue use the syntax:

SETPROP /3 queuename IDENTIFICATION "longname"

E.g.

SETPROP /3 FIRST_QUEUE IDENTIFICATION "PostScript printer in room 14"

To define standard parameters for a queue use the syntax:

SETPROP /3 queuename PARAMETERS "parameters"

E.g.

SETPROP /3 FIRST_QUEUE PARAMETERS "/NB /NT /NFF /TI=20"

Finally to disable (or hide) a queue from NiceCapt use the syntax:

SETPROP /3 queuename DISABLE_CAPTURE " "

You can view a setting by excluding the value parameter, hence

SETPROP /3 queuename IDENTIFICATION

SETPROP /3 queuename PARAMETERS

SETPROP /3 queuename DISABLE_CAPTURE

Finally you can remove a setting using the syntax:

SETPROP /3 queuename IDENTIFICATION -

SETPROP /3 queuename PARAMETERS -

SETPROP /3 queuename DISABLE_CAPTURE -

History

1.0

02/93	First realization of idea.

Problems :

Status didn't always show correctly.

1.1

01/94	Public release

Changes :

Improved stability and performance.

Problems :

Strange danish messages confusing everyone

Didn't always work with VLM's.

1.2

04/94	Didn't make it to the SimTel archives.

Changes :

Using new Netware library working with VLM's.

Fixes :

All messages translated into english.

Problems :

Reporting wrong captures in status window.

Not always checking all servers in VLM environment.

1.2a

05/94	Fixes :

Status window now shows the correct captures.

VLM environment problems fixed.

Future enhancements:

Help function (F1)

Add dialog box to view/change capture parameters.

Do the capture without calling capture.exe.