SEVENPAR

Calculation of seven parameters of the spatial Helmert transformation

SEVENPAR is a tool for the computation of highly exact transformation parameters of the spatial Helmert transformation, with which are possible precise coordinate transformations from one reference system into another. The determined transformation parameters can be entered directly into suitable transformation software (e.g. TRANSDAT Coordinate Transformation) or into the calculation software of GPS receivers to do exact coordinate transformations.

The seven transformation parameters are computed with a set of individual identical points. Those are points with known coordinates in two different reference systems. Cartesian coordinates or geographical coordinates with ellipsoidal height using the ellipsoid dimensions are suitable. Identical measuring points of higher order can be usually inquired or referred by the official institutions for measurement. The necessary ellipsoid half-axis are available in the program for the selection from lists.

The coordinates of the identical points are tested intensively on outliers and deviations by different statistic procedures. The quality of the computed seven Helmert parameters is documented by a residual matrix and by the computation of the middle, maximum and root mean square (RMS) deviations.

The seven parameters of the spatial Helmert transformation are calculated in the three most usual standards. Those are the Coordinate Frame Rotation, the Position Vector Transformation and the European Standard ISO 19111.

The network able program SEVENPAR is equipped with an up-to-date graphic user surface which handling can be understood intuitively. For each window detailed help texts can be shown. The user surface can be switched between German and English language.

Installation

The program SEVENPAR is stored in a directory of a CD ROM or on an other data carrier or is downloadable from the internet. In order to be able to use the program, it must be installed first.

The installation can be executed under Microsoft Windows 95 / 98, 2000, NT, XP, Vista and compatible operating systems.

For the installation of the program from a CD ROM or from other data carrier please execute the program SETUP.EXE, which is stored in the SEVENPAR directory of the data carrier.

After downloading from www and unpacking the file SEVENPAR.ZIP with assistance of PKUNZIP or WINZIP umpacking programs, please execute the program SETUP.EXE for the installing of SEVENPAR. PKUNZIP can be downloaded from our internet site.

It is important to close all open applications exept the windows explorer before starting the installation. Still open applications could use files, to which the installation program must have access during the installation.

Detailed Information about a network installation you can read after the here described installation on a network server in the electronic manual. The program supports Terminal Services on WINDOWS Servers like e.g. "Citrix Terminal Server".

Deinstallation

In the SEVENPAR start menue an icon is available for the complete deinstallation of the program.

Help

The help file contains global information to the program in form of an program intern electronic manual . The Englisch or German language help can be called from the SEVENPAR start menue or from the menue item "help" while the program is running.

Unlocking

Please read informations for the use of the free, but restricted test version and for the transfer of the test version into the unrestricted full version in the electronic manual.

Price list

Prices and a purchase order form for the order of the unrestricted full version of the program you will find in the SEVENPAR start menue or during the program is running under the menu item "help".

C. Killet Software Ing.-GbR

Postbox 400225

47896 Kempen

Germany

Fone: ++49 (0)2152  961127

Fax: ++49 (0)2152  961128

Email: killet@killetsoft.de

Internet: http://www.killetsoft.de