GX1/GX2/SCXX00
OSS LINUX
 Audio Driver

V 2.0.0

February 27th, 2004

Revision History

| 09/21/2000 -Version 1.0 | Initial Version by Mario Raposo |
| --- | --- |
| 02/05/2004 -Version 1.1 | Updated by Jens Altmann |
| 02/27/2004 -Version 1.2 | Changes for driver rel 2.0.0 |

Table of Contents

1. Overview/General Description	3

1.1 Target Platforms	3

1.2 Chipsets supported	Error! Bookmark not defined.

1.3 Firmware needed	Error! Bookmark not defined.

2. Description of Features	3

3. Preliminary tests	3

4. Design details	3

4.1 Gxaudio	Error! Bookmark not defined.

4.2 Duraudio interface	3

# Overview/General Description

## Target Platforms

- GX1

- GX2

- SCXX00

# Description of Features

- Playback of Audio files in all formats including mp3 files

- Recording of Audio files in all formats

- AC97 interface fully implemented

- Compatible with AD1819 CODEC

- Updated to 2.4.24 Kernel.

- simultaneous  playback and recording

# Preliminary tests

- gmplayer

- XMMS

- noatun

- gnome-sound-recorder

# Design details

## Gxlnxaud

This part of the driver takes care of the API calls and calls the appropriate hardware specific functions on the Duraudio interface. The picture below shows an overview of the driver in playback mode.
To decouple the user mode application, the data will be stored in a driver allocated buffer if no space is left
in the DMA space. The size of this buffer can be configured at compile time (default 512 kB).
The DMA buffer has a size of  only 64kB. (64kB plays just 50ms of  a stereo signal with 44100 khz sample rate and 16 bit resolution)  It cannot be guaranteed that the user mode application can be scheduled in such short time spans to reload the DMA buffer. For this reason the second buffer in the kernel has been implemented.

The next picture shows the data flow in record mode. There is no need to buffer received data in

the driver. The recording applications polls very fast the DMA buffer. If no data are available the read function  blocks until the next IRQ signals new data in the DMA buffer. A DMA buffer overflow has 
never been never observed even if a playback runs simultanously.

## Duraudio interface

It is an OS independent / NSC-hardware-specific interface. These functions can be used by any Operating system that is running our hardware.   They take care of all the communication to the audio hardware:

  - Find the PCI device

  - Prepare audio buffers to be read or filled with data

  - Set sample rates (DAC/ADC)

  - Handle interrupts

The function of this module has been described more in detail in DUAUDIO_specifications.doc

## Os_inc

Provides a set of OS related helper functions. These functions wrap the kernel API

And will be used by DURAUDIO and  Gxlnxaud.

- OS_ReadPortULong

- OS_ReadPortUShort

- OS_ReadPortUChar

- OS_WritePortULong

- OS_WritePortUShort

- OS_WritePortUChar

- OS_AllocateMemory

- OS_AllocateDMAMemory

- OS_FreeDMAMemory

- OS_CopyFromUser

- OS_CopyToUser

- OS_VirtualAddress

- OS_Get_F3BAR0_Virt

- OS_Free

- OS_Sleep

- OS_InitSpinLock

- OS_SpinLock

- OS_SpinUnlock

- OS_Free_F3BAR0_Virt

- OS_Free_VirtualAddress