PK232COM VERSION 1.41  PAGE 1

PK232COM Version 1.41 (c) 1987 by G3ZCZ

(C) Copyright 1987 - All Rights Reserved

By Joe Kasser G3ZCZ

POB 3419,

Silver Spring,

Md., 20901, USA.

PK232COM is written and supported by Joe Kasser, G3ZCZ.

The program is distributed as a Shareware product. You may freely

copy  and  share the product with your  friends,  associates  and

other radio hams. If you decide to use the product, you are asked

to  become  a  registered  user  by sending  a  QSL  card  and  a

contribution   to  the  author (suggested  amount  of  $35.00  or

equivalent in foreign currency.

The  contribution covers the cost of distribution of the updated

disk,  and  supports the continual development of "Software  for

Amateur  Radio".   Upon receipt of your contribution,  you  will

receive  one  free update disk),  mail (electronic  and  regular)

support,  and  notice of further releases.   The update disk will

contain  any  new versions of the software on this disk plus  any

new software, accompanying the series of articles currently being

written  on  the  applications of  data  processing  concepts  of

microcomputers to amateur radio.

Under  no circumstances may this product be sold or  distributed

with  another product without the express written permission  of

Joe Kasser, G3ZCZ.

Joe  Kasser,  G3ZCZ  will only support unmodified copies of  this

software.   Your comments and suggestions for changes are however

welcome.   If  you  are  the first to suggest a  change  that  is

implemented,  you  will be sent a complimentary copy of the  disk

with the change incorporated.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 2

CONTENTS

-------------

1.0  INTRODUCTION

1.1  PROGRAM HIGHLIGHTS

1.1.1     MORSE/BAUDOT/AMTOR/ASCII

1.1.2     PACKET RADIO

2.0  CRT DISPLAY

2.1  Status Display

2.2  Incoming Text Display

2.3  Outgoing Text Display

2.4  Command/Status Display.

3.0  Function keys.

3.1  Function Keys

3.1.1    Capture to Disk Toggle

3.1.2    Transmit a file

3.1.3    CAllsign Sequence.

3.1.4    Brag tape

3.1.5    Call CQ

3.1.6    Change Communications Modes

3.1.7    Call/Connect

3.1.8    Disconnect or Receive

3.1.9    Command Mode

3.1.10   Transmit Mode

3.2  Alt Function keys

3.2.1    Exit or return to DOS.

3.2.2    Initialize the PK-232.

3.2.3    Connect Status

3.2.4    Set Terminal Mode

3.2.4.1   SOLO

3.2.4.2   TRAFFIC

3.2.4.3   CQ/BEACON

3.2.4.4   READ THE MAIL

3.2.4.5   FEC

3.2.4.6   ALISTEN

3.2.4.7   ACHG

3.2.4.8   ARQ

3.2.4.9   Force LETTERS Case

3.2.4.10  Manual Mode

3.2.5    Monitor Channel Activity

3.2.6    Change Packet Connect / Contest / CQ text

Message

3.2.6.1 Change Packet Connect Message

3.2.6.2 Change Contest Message

3.2.6.3 Change CQ text Message

3.2.7    Enter Call Sign

3.2.8    Not used.

3.2.9    Activate Transmit Buffer

3.2.10   Help

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 3

3.3  Shift Function Keys

3.3.1     Dir

3.3.2     Rename

3.3.3     List of Outgoing Messages

3.3.4     Delete

3.3.5     List of messages that have been sent.

3.3.6     Mail Beacon Activate

3.3.7     Retry a call/connect request

3.3.8     Not used.

3.3.9     Connected Indicator Reset.

3.3.10    OVER function.

3.4  Control Function Keys

3.4.1     Day Flag

3.4.2     CWID

3.4.3     Message Uploading

3.4.3.1   Leaving a Note in your system.

3.4.3.2   Leaving  a message on an  other

PK232COM system [QSP].

3.4.3.3   Uploading  messages to  another

PK232COM system.

3.4.4     Show last few log entries.

3.4.5     Automatic CQ Sender

3.4.6     SELCAL

3.4.7     Beacon  Activate  Toggle/CQ  Wait  Delay

Adjust.

3.4.7.1   Beacon Mode Toggle/On-Off

3.4.7.2   CQ Delay Time Adjust

3.4.8     Not used.

3.4.9     Transmit Buffer Clear

3.4.10    Log QSO

3.5  Shift/Baudrate Control keys

3.5.1     The RIGHT ARROW Key (Increase Baudrate).

3.5.2     The LEFT ARROW Key (Decrease Baudrate).

3.5.3     The UP ARROW Key (Change shift).

3.5.4     The DELETE Key (toggle USOS).

3.6  Additional keys

3.6.1     The HOME Key.

3.6.2     The DOWN ARROW key.

3.6.3     The Alt D key (display inhibit).

3.6.4     The Alt P key (printer toggle).

3.6.5     The Alt F key (Digipeat monitor toggle).

3.6.6     The Alt R key (Message Relay toggle).

4.0  PK232COM disk files

4.1  PK232COM.COM

4.2  PK232COM.RUN

4.3  PK232COM.SYS

4.3.1     The first line (callsign).

4.3.2     The second line (AMTOR SELCAL).

4.3.3     The third line (CQ Text).

4.3.4     The fourth and fifth lines (Band and Power).

4.3.5     The sixth line (CQ Delay time).

4.3.6     The seventh line (Message Relay Capability).

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 4

4.3.7     The eight line (Communications Port).

4.3.8     The ninth line (TNC type)

4.3.9     The remaining lines.

4.4  PK232COM.LOG

4.5  PK232COM.DIR

4.6  CONFIG.SYS

4.7  *.MBX

5.0  Initialization

6.0  Running the Program

7.0  Mailbox features (AMTOR/Packet Modes Only)

7.1  Answering Machine.

7.2  Mail Beacon (Annunciator).

7.3  How to leave a message

7.4  Remote File Downloading (Packet Mode Only).

7.5  Remote Beacon shutdown.

APPENDIX 0. UPDATES/REVISION HISTORY

APPENDIX 1. BUGS

APPENDIX 2. MESSAGES IN A LOCAL AREA NETWORK

APPENDIX 3  BASIC NC/L DICTIONARY

APPENDIX 4. USING OTHER TNC'S.

4.1 TNC2 (MFJ 1274)

APPENDIX 5. NOTES

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 5

1.0  INTRODUCTION

PK232COM  is a terminal interface program for the PK-232.   It is

designed  only  for  the PK-232  built  by  Advanced  Electronics

Applications  Inc.  and not for just any TNC.  Most of the packet

radio  features however should work on any TNC.   It is  designed

to  allow  anyone to use and get the most out of the  PK-232  for

Morse,  ASCII,  BAUDOT,  AMTOR  and  Packet Radio  communications

without having to keep the PK-232 manual handy.

PK232COM  is  a  sophisticated program.   In its basic  state  it

allows you to use the PK232 in an optimal manner.   It configures

the PK232 for you to maximise the communiations efficiency in the

communications mode of your choice.  That means for example, when

working Packet on HF you need to program the PK232 parameters  to

different values than you would use on VHF to make maximum use of

the  mode (one significant difference is the length of the packet

itself,  for the longer it is, the greater the probability of QRM

destroying  it).  This program will adjust the packet  parameters

for you.

Since  the  computer is now involved,  other features  have  been

added  to  simplify operation,  and several  features  have  been

automated.   All  these  operations are performed using  function

keys as documented below.   It will take a while to learn how  to

use  this program in a manner that suits you,  read this document

and  have  fun,  after all,  isn't that one of  the  purposes  of

Amateur Radio.

This  documentation  file is not a manual.  It is  a  preliminary

document.   It  is written to remind you of what the program  can

do.   If  there is enough interest (registrations) I will  expand

it.   It  is  not  designed  to explain  the  theory  behind  the

different communications modes.   Read the PK232 Manual for  more

information.    This   program   is  designed  to  optimize   the

configuration  of  the PK232 in each communications mode  and  to

provide some smart terminal features.

1.1  PROGRAM HIGHLIGHTS

1.1.1     MORSE/BAUDOT/AMTOR/ASCII

Automatic  CQ  caller.   Will call CQ repetitively  and

signal you when a reply is received.

SELCAL.

Automatic  capture to disk of all ingoing and  outgoing

text of any QSO.

1.1.2     PACKET RADIO

Automatic  disk recording of all packet radio connects.

Automatic  indication of Packet connects.   Local  Area

Network message store and forward.  Digipeat monitoring

and capture.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 6

AMTOR/PACKET  Automatic Answering Machine capability with display

of message queue.

Automatic   Beacon   Mode   CQ  caller  in   Packet   and   AMTOR

communications modes.   Will call CQ repetitively and either work

the connect and keep going after disconnect or signal you when  a

reply is received.

Automatic  logbook  entries  for  Packet and  Beacon  mode  AMTOR

Connects,  semi automatic logbook entries for other  modes.   Log

file  is  compatible  to  that  of the  CONTEST  program  in  the

CONTEST.ARC  archive and can be processed by the  DBASE   Logbook

Package of PC-HAM by G3ZCZ for indexed listings, tracking of DXCC

and other AWARDS, etc.

Contest  operation,   sends  standard  mesage  and  automatically

increments QSO count.

Automatic   optimized   configuration   of  the  TNC   for   each

communications mode.

Morse,  ASCII,  BAUDOT communication modes are HALF DUPLEX.   One

station  is transmitting at any time.   You thus have to use  the

Transmit/Converse  and Receive/Disconnect function keys  to  turn

the transmitter on and off respectively.

AMTOR  is also HALF DUPLEX but has some of the attributes of full

duplex.   In  this  mode  you  control  who  is  talking  through

software,  only one station can send data at a time,  but you can

type  characters in  to the buffer and they will  be  transmitted

automatically  when the other station turns the QSO over to  you.

Once   you  have  linked  with  someone,   you  do  not  use  the

Receive/Disconnect function UNTIL THE QSO IS OVER.  Both stations

are communicating automatically thanks to the TNC.   If you  want

the  other operator to respond type the +?  key sequence (or  use

shift function key 10).

Packet  Radio communications  on the other hand is a full  duplex

mode  of communications.   Once you have connected with  someone,

you  do not use the Receive/Disconnect function UNTIL THE QSO  IS

OVER.   Both  stations are communicating automatically thanks  to

the  TNC.   If you want the other operator to respond type the  >

key followed by the 'Enter' key (or use shift function key 10).

Note  that  Morse,  AMTOR,  ASCII and BAUDOT are  character  mode

communications  in  that each character you type is sent  as  you

type them.   In Packet Communications, the TNC stores them up and

sends out a burst or packet when you type the 'Enter' key or  you

type  enough  characters to fill up a  packet.   Therefore  don't

forget to use the  'Enter' frequently, especially before you stop

to think about what to send next.

Any  Terminal  Node  Controller  (TNC)  is  a  complex  piece  of

equipment.   It can operate in a Command Mode,  in which you tell

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 7

it to do something,  or in a Converse Mode in which you are using

it  to talk to other stations.   Many people confuse the two when

first  getting on Packet.   If you monitor the channels you  will

recognize command mode TNC instructions on the air,  and when you

use  the  TNC you will receive the "?What" reply  when  you  type

something  thinking  that  you are in the Converse Mode  but  are

really in the Command Mode.   PK232COM is designed to make Packet

operation simple using high level (Function key) commands.

Version  1.0 has the following functions built in to it in a menu

driven format.

Morse Code  20 wpm (CW).

BAUDOT  with default of 45 WPM narrow shift (LSB).   You may  use

the  left  and  right  arrow keys to change the  baud  rate  (see

below).

ASCII 110 BAUDS narrow shift (LSB).

Packet VHF or HF baud rates and shifts (LSB).

AMTOR 100 BAUDS narrow shift (LSB).

All  communications  operate in the 7 bit or normal  ASCII  mode.

You  cannot  send  8 bit files (compiled  programs,  etc.)  using

Version 1.XX of PK232COM.

Access  to  the PK-232 Command mode is provided in case the  user

wishes  to  override  any  of these defaults  EXCEPT  the  7  bit

communications mode.

IF YOU USE THIS PROGRAM, YOU STILL MUST READ THE PK-232 MANUAL.

Note   that   AMTOR   and  PACKET  are  full  duplex   modes   of

communications.   When you connect with someone, there is no need

to use the transmit and receive function keys.   In fact you will

have  problems  if you do.   Do not disconnect until the  QSO  is

over.

WARNING  ....   THIS PROGRAM OPERATES IN AN OPEN LOOP  MODE.   IT

REMEMBERS WHAT INSTRUCTIONS OR COMMANDS IT GAVE THE TNC.   IF FOR

SOME REASON,  THE TNC IS NOT SYNCHRONIZED TO THE PROGRAM YOU WILL

NOT GET CORRECT DISPLAYS ON THE SCREEN.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 8

2.0  CRT DISPLAY

The CRT display contains four areas as follows.

-------------------------------------------------

|             STATUS DISPLAY                    |

-------------------------------------------------

|                                               |

|                                               |

|             INCOMING TEXT DISPLAY             |

|                                               |

|                                               |

|                                               |

-------------------------------------------------

|             OUTGOING TEXT DISPLAY             |

|        ____________________________           |

|       |____________________________|          |

|                                               |

-------------------------------------------------

2.1  Status Display

A  one  line display at the top of the  screen  showing  the

status of the communications system.

The  left side of the screen normally displays the  "capture

to  disk"  or  logging status,  the  center  of  the  screen

displays  status  information  depending  on  which  of  the

operational modes is active.

2.2  Incoming Text Display

The  center section of the screen displays data coming  back

from  the PK-232.   This consists of data received  via  the

radio  communications  link,  PK-232 status information  and

transmitted data echoed back by the PK-232.

2.3  Outgoing Text Display

The  lower area of the screen shows data being sent  to  the

PK-232.   This  consists of command data from the program to

the  PK-232,   text  typed  by  the  user  and  files  being

transmitted.

2.4  Command/Status Display.

This  screen  located above the Outgoing Text Display  shows

the  command dialog between the program (PK232COM)  and  the

PK232.   It is displayed for educational use so that you can

see how the PK232 is being programmed.

3.0  Function keys.

The operation of each of the function keys are explained below.

3.1  Function Keys

3.1.1    Capture to Disk Toggle

This  key toggles the "capture to disk"  function.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 9

When  the  function is active,  all incoming  text

(including 'Echo as sent' outgoing text) is stored

on  a  disk  (floppy or hard)  in  a  file  called

'PK232COM.RUN'.   If  this file does not exist the

first time that the function is activated,  it  is

created.     From   that   time   on,   successive

activations append new data to the contents of the

file.

Each  time  the function is activated a  datestamp

will be written to the file.

When  a  Packet station connects the  'capture  to

disk'  function  will be  activated  automatically

even  if  you  have  not  set  it  active.  Packet

connects  will thus be recorded in their  entirety

automatically  unless  you turn  the  logging  off

during the course of the QSO. The function will be

deactivated  at  disconnect time unless  you  have

toggled it on.

You  may  later edit the file at your  convenience

with your favorite word processor.

The function will automatically self terminate and

close  the  log  capture file  if  you  forget  to

deactivate  the  function before  terminating  the

program and returning to DOS.

3.1.2    Transmit a file

This function allows you to transmit an ASCII file

via  the PK-232.   You may abort the  transmission

sequence  at  any  time by depressing any  of  the

function keys,  the keys on or associated with the

numerical pad or the ESC key.

While the file is being sent, you will see various

things on your screens.  The text of the file will

be seen in the Input window as it is being sent to

the TNC.

During the file transfer,  the status window  will

show the different states of the transmission.  It

begins  with  a signal that the message  is  being

sent.   If  the  text  is long and fills  the  TNC

buffer,  you will see a message 'QRX PK232  BUFFER

FULL'    possibly   followed   by    'PK232    NOT

RESPONDING'.  This  latter  message will  seem  to

blink periodically.   When the PK232 signals  that

it  is  ready  to accept more  text,  the  message

sequence will resume.

When  the file has been loaded into the PK-232 the

computer will add the ^Z character to the outgoing

text  to  signal to the recipient  that  the  file

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 10

transmission  is  over.   You may abort  the  file

transmission  sequence by depressing any  function

key,the  'ESC' key or any other Control or  Alter-

nate key on the keyboard.

The  computer  will  signal you  (using  the  bell

character,  when the file has been loaded into the

PK-232.   Note  that is not the same as  the  time

that the PK-232 has transmitted the file,  because

the PK-232 incorporates a character buffer.

3.1.3   Call Sign Sequence.

This  function  lets  you transmit  the  following

sequence <other call) DE <your call>.  For example

if  you are 4X6AA and you are working  G3ZCZ  then

this function would transmit the sequence G3ZCZ de

4X6AA.    You   use   this  at  the  start  of   a

transmission in any mode except Packet.

3.1.4    Brag tape

This  function  allows you to transmit a  standard

text file historically known as a "Brag Tape"  via

the  PK-232.   The  brag tape file must be  called

PK232COM.TXT,  and  should be created with a  word

processor in the non-document mode.  A sample file

is  included on this disk.   Transmission  of  the

Brag  Tape may aborted in a similar manner to that

of a regular file (see above).

3.1.5    Call CQ

You  use  this function key to call  CQ.   In  the

Packet  Communications  mode,   the  PK-232   will

transmit  a line of text along with the CQ packet.

The  line  of text is set up in  the  PK232COM.DAT

file described in section 4.

In the AMTOR communications mode, your SELCAL will

be  transmitted along with the CQ call in the  FEC

mode.

3.1.6    Change Communications Modes

This   function   allows   you   to   change   the

communications  mode  of  the  PK-232.   Allowable

modes are Morse code, BAUDOT, AMTOR standby, AMTOR

Alisten,  ASCII Packet and Signal.   In the Packet

mode  you  can  choose VHF or HF  operation  in  a

single step by entering the letters V or H as well

as  the letter P for packet which will take you to

a second prompt requesting VHF or HF.

When  you  select the AMTOR standby mode  you  are

placed  into the 'standby receive' mode.   If  you

want to read the mail on an existing AMTOR QSO  in

progress,  select  the AMTOR Alisten or  chirpcopy

mode.   If  you are in one mode,  you will not  be

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 11

able to see signals of another mode.

Use  the 'G' option to select the  'SIGNAL'  mode.

Read   the   PK232  documentation   carefully   to

understand  how this mode works.   Once the  PK232

has identified the digital signal it is tuned  to,

use the same function key a second time to  select

the relevant mode and set the baud rates and shift

to display the data.

3.1.7    Call/Connect

You  use this function to call a station heard  on

the  band,  or to attempt a connect  with  another

packet station.  You may also use it to call 'QRZ'

or even 'TEST'.

In  AMTOR the call will be sent in the ARQ mode to

the SELCAL of the desired station.

In the Packet Communications mode you can create a

directory  file  and  keep  a list  of  names  and

callsigns.   The computer will look up a name  and

call the station by its callsign.   For example if

you  put

Joe G3ZCZ

as a line in the file,  when you tell the computer

to  connect  to  Joe,  it will try to  connect  to

G3ZCZ.   See  section  4.5  which  describes   the

PK232COM.DIR file for further information.

In the Packet communications mode,  if you want to

loop  back  through  someone else  enter  the  '/'

character  followed by the call of the station you

want  to  loop back through.   For  example  if  I

(G3ZCZ)  want to loop back through 4X6AA,  I  just

need to enter /4X6AA as the call to connect to and

the  computer will tell the TNC to try to  connect

to G3ZCZ via 4X6AA.

3.1.8    Disconnect or Receive

This  function  key  returns the  station  to  the

receive communications (PK232 command)  mode.   It

may  also  be  used  to  abort  the  automatic  CQ

sequence.   If  you use it to abort a CQ  sequence

while the transmission is in progress,  flush  the

PK232  transmit buffer using Control function  key

9.

3.1.9    Command Mode

This  function  puts the PK-232 into  the  command

mode.   At  this time,  the program is transparent

and  acts as a dumb terminal (except  for  placing

incoming   and  outgoing  text  in  the   relevant

windows).  you use this mode for overriding any of

the  PK-232 default conditions.   (SEE I TOLD  YOU

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 12

THAT YOU HAD TO READ THE PK-232 MANUAL).

3.1.10   Transmit Mode

This  function places the PK-232 into the transmit

or  converse mode.   The transmitter is turned  on

and  the  contents  of  the  transmit  buffer   is

transmitted.   In  the  non packet  communications

modes, the outgoing characters will be echoed back

from  the  PK-232 and displayed  in  the  INCOMING

window  on the CRT as they are transmitted on  the

air.

3.2  Alt Function keys

3.2.1    Exit or return to DOS.

Exits  the program and returns you to  DOS.   When

you exit the program,  the TNC is put into the VHF

default  mode in which it rejects calls  from  any

other  stations.   (Read about the 'CFROM' command

in  the  TNC manual).   you can thus if  you  wish

leave  it powered,  and if you also leave the  VHF

radio  powered  up,   you  will  be  active  as  a

digipeater,  but at the same time anyone trying to

connect with you will receive a busy signal.   The

MBX function (Reading the Mail) is also cleared at

this time.

3.2.2    Initialize the PK-232.

Sets up the defaults for the PK-232 from the  file

PK232COM.SYS.   You  should normally only have  to

run  this  function when you first  power  up  the

PK232 or change the batteries.

3.2.3    Connect Status

When  you  are in the Packet communications  mode,

this  function  lets  you see  the  state  of  the

connection  in the Command Window.   It will  show

you  if  there are any unacknowledged  packets  en

route  to  the other station.   If you  have  been

sending text, it is advisable to use this function

before you disconnect, just to ensure that all the

data  has  been received and acknowledged  by  the

other party.   You may use this function in either

the TNC Command or the TNC Converse mode, but note

that  using this function will set the TNC to  the

Command Mode.

3.2.4    Set Terminal Mode

This   key   allows  you  to  set   the   terminal

configuration mode.   In the PACKET Communications

mode, the function works in the following manner.

After setting the terminal configuration mode,  it

returns  you to the TNC command mode.   In  Packet

communications  many stations can use the  channel

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 13

at  the  same time.   Refer to the  "MONITOR'  and

'MBX' commands in the PK232 manual.

You can set four Packet terminal modes as follows.

3.2.4.1   SOLO

In  this  mode,  you will only  see  messages

addressed to you.  You will only get messages

from  people  who  connect  to   you.   (This

corresponds to 'MONITOR 0').

3.2.4.2   TRAFFIC

In this mode you will see most of the traffic

on  channel.   you can use this mode to check

that the TNC is working. (This corresponds to

'MONITOR  4').

3.2.4.3   CQ/BEACON

In  This  mode,  you will see CQ  and  BEACON

packets on the channel.  (This corresponds to

'MONITOR 1').

3.2.4.4   READ THE MAIL

Some  packet channels are busy.   To cut down

on  the  number  of  messages  sent  on   the

channel,  you  can read the mail on a BBS  or

other   station  and  get  bulletins  without

connecting to that station yourself.  You use

this mode to do so.  you will be prompted for

the  call whose packets you wish to  monitor.

Remember  to  enter the call of  the  station

sending the packets,  not the call of the one

receiving   them.    (This   corresponds   to

'MONITOR  0'  and 'MBX' callsign).

You  can  also use this function  to  monitor

both sides (read the mail) on a QSO on a busy

packet channel.

You  may  want  to  turn  the  Digipeat  Data

Display mode off when reading the mail.

The  Packet  Terminal  mode of operation  will  be

shown  in  the  Status window at the  top  of  the

screen.

If  you  are reading the mail on  a  station,  the

callsign of that station will be shown next to the

word "MAIL".  If you are connected to someone, the

call (and path) will be shown prefixed by the "*>"

sequence.

You  also use this function to go into and out  of

the multiple connect mode.   The default condition

is one connect at a time.   Should you opt for the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 14

multiple connect mode, make sure you know what you

are doing, and have set the CHSWITCH character.  I

recommend  using the '\' (backslash) but if you do

use it,  don't transmit raw directory files in the

mailbox  file  mode  or  you  will  see  an  error

message.  You don't have to be in the command mode

to  use the CHSWITCH character.   Read  all  about

CHSWITCH in the TNC manual.

When  you  are in the multiple  connect  mode,  an

asterisk  will be shown in the status window  next

to the word 'PACKET'.

In  the AMTOR communications mode you can also  do

four things as follows;

3.2.4.5   FEC

Places  the  terminal into the  FEC  transmit

mode.  This puts you on the air, anything you

type will be transmitted.

3.2.4.6   ALISTEN

Read the Mail on a QSO.  Unfortunately,  when

you  tune in a chirpy AMTOR QSO in the ARQ or

standby  mode you won't get any copy  on  the

QSO  in progress.   You can however read  the

mail on one by entering the ALISTEN mode.  If

you  want to see a FEC CQ call you will  have

to return to the ARQ mode.

3.2.4.7   ACHG

Force a changeover or 'break in'.  If for any

reason   you  want  to  interrupt  the  other

station you send a 'BREAK' signal in software

this way.

3.2.4.8   ARQ

Return to the ARQ standby mode.  You must  be

in  this mode to receive signals from another

station  who is calling you.  You may  do  it

this  way or via the main Mode   Menu.   When

you  tune for CQ calls,  they sound  somewhat

like  fast RTTY with a ringing sound  similar

to the connect alarm (but at a faster ringing

rate) used by PK232COM.

3.2.4.9   Force LETTERS Case

This  function lets you force the receiver to

the  letters  case  if for  some  reason  you

appear  to be in the numbers case.   It  will

return you to the converse mode.

3.2.4.10  Override ROBOT Beacon

This  function  lets you override  the  ROBOT

BEACON  and  chat  to  a  station  that   has

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 15

linked  to  the  ROBOT.   Use  it  after  the

message  in  PK232COM.TXT has been  sent,  ie

when  the other station is sending a  message

to you.

3.2.5    Monitor Channel Activity

This  function  lets you see what calls the  PK232

has  heard recently on the Packet  channels.   You

can see up to 18, with the data and time they were

last  logged  by the PK232.   If they  were  heard

directly  an asterisk (*) will be listed  next  to

the   call  sign.    If  they  were  heard   being

re  transmitted via another station,  the asterisk

(*) will not be shown.   Refer to the 'MH' command

in the PK232 manual.

3.2.6    Change Packet Connect/Contest Message

3.2.6.1   Change Packet Connect Message

This   function  allows  you  to  change  the

message  that  will be  received  by  another

packet station when it connects to you. Refer

to the 'CTEXT' command in the PK232 manual.

3.2.6.2 Change Contest Message

This function first toggles the contest mode.

If  the  contest mode  is  active,  the  word

'TEST' and the number of the next QSO will be

displayed  in  the status  window.  When  the

contest  mode  is toggles on,  this  function

then  allows  you  to set  the  starting  QSO

number  and  enter  in the  non  changing  or

standard message that will be transmitted  as

part  of  an  exchange of messages  during  a

contest.

The contest function is toggled off the  next

time  you select this function,  or when  you

change  the  PK232 Communications  mode  (ie.

switch from Morse to BAUDOT).

Note  that while this does give you some semi

automatic contest operation by using function

keys,  there is no checking of callsigns  for

duplicates  built  into the  program.   In  a

contest,  you  enter the callsigns  and  call

the other station in the normal manner,  then

use   this  function  key  to  transmit   the

exchange.  In Morse, and RTTY don't forget to

use function key 10 to turn on the transmiter

first.

3.2.6.3 Change CQ text Message

This  function  allows  you  to  change   the

contents of the CQ text message string.  This

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 16

is  the string that is transmitted along with

the  CQ call.   The default value  is  loaded

from the PK232COM.SYS file.

3.2.7    Enter Call Sign

This  key lets you enter the call of  any  station

calling  you  into the program.  If you  type  the

enter key in response to the prompt, you can later

use  the  F3  (function key  3)  to  transmit  the

sequence "de <yourcall>".

In AMTOR,  you use it to enter the callsign of the

station you want to or have called by means of the

SELCAL.

3.2.8    Not used.

You  don't want to disconnect accidentally if  you

just depress function key 8 without the 'Alt' key.

3.2.9    Activate Transmit Buffer

The  use  of this key sets PK232COM to  the  "Type

Ahead"  mode.   Any characters then typed  at  the

keyboard  will be placed into the PK-232  transmit

buffer.  The contents of the buffer will be trans-

mitted  over the air when the transmit function is

invoked  [FK10].   THIS FUNCTION IS NOT ACTIVE  IN

THE PACKET MODE since Packet is a full duplex mode

of communications.

This feature allows you to 'type-ahead' as you can

enter text into the transmit buffer while  viewing

incoming words in the INPUT window in the  screen.

Any  characters  typed from the keyboard  will  be

echoed in the OUTGOING window on the screen.

3.2.10   Help

The use of this key will activate the HELP menu in

a special window on the screen.   This window will

remain  on  display until a character is typed  at

the  keyboard,  or a character is received by  the

computer from the PK-232.

3.3  Shift Function Keys

3.3.1     Dir

This  function  shows  you the  directory  of  the

logged in (active) disk drive.

3.3.2     Rename

This  function allows you to rename a  file.   You

could  for  example rename a document  file  to  a

"callsign.OUT"  file so that someone will  receive

it automatically next time they connect.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 17

3.3.3     List of Outgoing Messages

This function shows you the outgoing messages that

have not been sent. In other words, all files with

the  file  extension  of '.OUT'.See  below  for  a

description  of outgoing messages in your  "Packet

Radio Answering Machine".

3.3.4     Delete

This function allows you to delete a file

3.3.5     List of messages that have been sent.

This  function  shows you the messages  that  have

been  sent.   In other words,  all files with  the

file extension of '.OLD'.

3.3.6     Mail Beacon Activate

Using this function will also load a 'Mail :' list

into  your  Packet communications mode Beacon  and

initiate  a  beacon transmission every 30  minutes

(Refer  to the BTEXT command in the  TNC  manual).

If  no  mail is pending then beacon  transmissions

are  inhibited.   This conforms to good  operating

practice on crowded Packet channels.

3.3.7     Retry

This  function allows you to retry a connect  that

for some reason did not go through the first time.

It save wear and tear on your fingers particularly

if  the  desired connect path is via a  number  of

relays.

3.3.8     Not used.

You  don't want to disconnect accidentally if  you

just  depress  function key 8 without the  'Shift'

key.

3.3.9 Connected Indicator Reset.

You  use this function key to reset the PACKET and

AMTOR   beacon  mode  "connect  has  taken   place

indicator".  This is a flashing [C] display in the

status window.   Whenever someone else connects to

you this display will be activated.   It is  reset

by  this  function  key.   If you  are  monitoring

Packet  traffic on a channel,  you may not  notice

that a connect has previously taken  place.   This

indicator  alerts you to check the contents of the

PK232COM.RUN disk file for any incoming  messages.

The  number  shown with the 'C' is the  number  of

connects that have taken place since the Indicator

was last reset.

3.3.10  OVER function.

In  the Packet communications mode  this  function

sends  your  callsign followed by the >  and  Line

feed  characters.   You  may  use this  to  signal

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 18

'OVER' in a packet contact.

In  the  AMTOR communications mode  this  function

sends  a  callsign sequence followed by  the  '+?'

sequence  of  characters  which are  the  software

'OVER' signal.

In   the   MORSE  and  RTTY  (Baudot  and   ASCII)

communications  modes,   this  function  lets  you

transmit  the following sequence <other  call)  DE

<your  call> and returns you to the PK232  Command

Mode.   For  example if you are 4X6AA and you  are

working  G3ZCZ  then this function would  transmit

the sequence G3ZCZ de 4X6AA.   You use this at the

start of a transmission in any mode except Packet.

If  you have typed something into the 'type  ahead

buffer' then touch this function,  the contents of

the buffer first will be transmitted and that will

then be followed by the callsign sequence.

3.4  Control Function Keys

3.4.1     Day Flag

This  function key puts the TNC into  the  command

mode and loads the date into it from the computer.

You  use  this to reset the TNC clock if it  isn't

that stabel,  or to flag a date and time into  the

log file when in the non Packet modes. For example

if you want to time tag a qso.

3.4.2     CWID

This   function   key  is  used  to  send   a   CW

identification  when  operating BAUDOT,  ASCII  or

AMTOR.   You use this if your license requires you

to  identify  in  morse code as  well  as  in  the

operating mode.  In AMTOR, the CWID is sent in the

FEC  mode.

The  identification consists of the  sequence  'DE

<your  callsign>'  where  your call sign  is  that

entered into the PK232 as  'MYCALL'.

3.4.3     Message Upload

PK232COM  contains store and forward routines  for

messages   in  a  Local  Area  Network  (LAN)   as

described  in Appendix 2.   Use this  function  to

upload a message to another system compatable with

PK232COM.

3.4.3.1   Leaving a Note in your system.

In all modes you can leave a brief  note

(message) for another station.  When you

invoke this function the header is writ-

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 19

ten  to  the message file,  then as  you

enter the text it is stored on the  disk

in the usual format.

WHILE WRITING YOUR NOTE,  ALL OTHER FUN-

CTIONS  OF  PK232COM  ARE  INHIBITED  OR

LOCKED OUT.  THUS IF SOMEONE CONNECTS TO

YOU,  THE TNC WILL KNOW THT BUT PK232COM

WILL NOT.  BE WARNED !!!

When  you invoke this function in the Packet  mode

While  connected  to another  station.   When  you

invoke this function, the computer will ask you to

enter if you want to QSP or QRU.

3.4.3.2    Leaving a message on an other  PK232COM

system [QSP].

The  QSP  function lets  you  enter  the

callsign  of  the  station you  wish  to

leave a message for.   It will then send

the text sequence :QSP: <callsign>.  The

other  station will either  reply  :QNO:

followed  by  a number  or  :QRV:  <cal-

lsign>.   The  'QNO' message means  that

the  other  computer cannot  accept  the

message, the :QRV: message means that it

is waiting for you to send it.

If you get the :QRV: reply, go ahead and

type the message,  or send the text file

containing the message.  If you type the

message,  use a control Z (^Z) character

and  carriage  return to  terminate  the

message.  You will then receive either a

:QSL: frm the other station meaning that

the  message  is accepted,  or  a  :QNO:

followed  by  a number which means  that

something went wrong and the message was

not stored.

3.4.3.3    Uploading messages to another  PK232COM

system.

The QRU function allows you to upload  a

number of messages to the other station.

You   use  it  when  you  have  messages

pending on your system and want to  pass

them  to a third computer for storage in

the  network,  while you take your  com-

puter off-line.

You  can only use this function  with  a

station  designated as a store and  for-

ward mail box, for which you have set up

a  <callsign>.MBX file (see section  7).

This  function  may be triggered by  you

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 20

using   this  control  key  or  by   the

connecting  station sending you a  :QRU:

command.

When the sequence is over,  the computer

receiving the messages will transmit the

command  word :QJG:  to signal that  the

sequence   is  complete.    Should   you

receive  it before you get any messages,

then there weren't any pending.

If  you  want to download messages  from

another  PK232COM system connect  to  it

and transmit the sequence :QRU: followed

by  at least one space character.   That

system  will then respond to you  as  if

the  operator there had initiated a  QRU

function.

Appendix  2 contains more information on  the  LAN

capabilities of PK232COM.

3.4.4     Show last few log entries.

This function displays the log entries made during

the  active  session.  If no QSO's have been  made

since  the program was invoked the display  window

will  be  blank.   Up  to 5 log  entries  will  be

displayed  in the window.   You use this  function

for  a quick look at which stations have connected

to you in your absence.

3.4.5     Automatic CQ Sender

This function will send a CQ sequence,  wait about

a  minute  or so  and then repeat the CQ  sequence

every  minute  or so until it receives  your  call

back from someone else.   When it does so, it will

alert you with a Connect Alarm signal and reset to

the normal or listening mode.

When the Automatic CQ sequence is in operation, an

indication  will  be shown in the  status  window.

The display will be constant while the transmitter

is  sending the CQ test,  and will blink while the

receiver is listening for a reply.

The  automatic CQ function will only work  in  the

non  Packet modes if the 'EAS' mode is set to 'ON'

per the PK232COM.SYS file.   In other  words,  you

must  be  able  to see the CQ text pop up  in  the

outgoing  window  and  then  slowly  echo  in  the

incoming  window for the function  to  work.   The

reason  for  this is that the "K K K" sequence  is

used  to  arm  the  software  to  look  for   your

callsign.    If  it  wasn't,   the  program  would

recognize your call being echoed to the screen and

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 21

think that someone had called you.

In AMTOR you will get the connect alarm only  when

the  replying station transmits the '+?' character

combination.

The  check for the detection of your callsign  and

the connect alarm sequence only occur after a line

feed character have been received.

Under most conditions in AMTOR,  many in RTTY, and

some  in  MORSE,   PK232COM  will  recognize   the

callsign  of  the  station answering your  CQ  and

display  it in the status window.   In  the  event

that  it fails completely,  the  sequence  '--X--'

will  show  up in the status  window.   Since  the

recognition  algorithm use just look for the first

word following the 'DE ' sequence and assumes that

it has the callsign,  be careful to check it  when

operating under QRM conditions.   You can override

the  automatic call selection with the Enter Call-

sign function key.

The  automatic  CQ sequence is cancelled  when  an

incoming call is detected, you push the disconnect

function  key,  or  if you  change  communications

modes.

3.4.6     SELCAL

This  function  key toggles the SELCAL feature  on

and off.   When the SELCAL is activated,  as shown

in the status window,  the incoming text from  the

PK232 is scanned.   When either the letters "ZCZC"

or 'your callsign' (as entered in the PK232COM.SYS

file),  the  disk  recording is  turned  on.   All

subsequent text will be stored in the PK232COM.RUN

disk  file until the letters "NNNN" are  received.

At that time the disk recording is turned off.

You   may   use  this  feature   when   monitoring

commercial  RTTY  broadcasts  to record  news  and

other  information,  or when calling CQ  to  start

recording  all  incoming text and keep the QSO  on

disk.

You may use the loggletoggle function to  override

the automatic turnoff if you wish.

3.4.7     Beacon Toggle On/Off/CQ delay time set.

This  function toggles the beacon option  for  the

automatic  CQ  caller  in  the  AMTOR  and  Packet

communications modes.  When active and CQ is being

called,  a  [B]  will be displayed in  the  status

window.  In  this mode,  the Automatic CQ sequence

will  resume after the AMTOR link is dropped or  a

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 22

Packet Communications mode disconnect.

In the beacon mode,  the last few logbook  entries

will  be  displayed  in the logpage  window  which

overlays the outgoing text window at the bottom of

the screen.

In the Packet mode,  the CTEXT message is the only

transmission from you that the other station  will

receive, so make it good.

In   the   AMTOR  mode,   the  contents   of   the

PK232COM.TXT  file  will  be  transmitted  to  the

station  who  called you.   The disk capture  will

then be turned on to record one transmission  from

that station.   The QSO will then be logged to the

logbook  file.   Don't forget to check the entries

in case the computer made a mistake in recognizing

the call sign of the other station.

THIS  MODE IS DESIGNED TO ENCOURAGE PEOPLE TO  QSY

OFF THE CROWDED PACKET HF CHANNELS.  DO NOT USE IT

ON  A  CROWDED  HF CHANNEL OR  ON  AN  ACTIVE  VHF

CHANNEL.  IF YOU DO, YOU WILL SOON FIND THAT OTHER

STATIONS  HAVE  SET  THEIR TNC'S  TO  REJECT  YOUR

PACKETS,  YOU WILL NOT WORK ANYBODY, YOU WILL JUST

BA A SOURCE OF QRM AND WILL BE KNOWN AS SUCH.

THE  MODE IS ALSO DESIGNED FOR ROBOT  BEACONS  FOR

THE HIGHER FREQUENCIES.   FOR EXAMPLE IF YOU LEAVE

IT  RUNNING  ON  10 METERS YOU NEVER KNOW  WHEN  A

SHORT OPENING WILL ALLOW A QSO TO SOME PART OF THE

WORLD.   A DX CONTACT YOU WOULD HAVE MISSED OTHER-

WISE.    DO NOT LEAVE IT RUNNING ON 20 METERS WHEN

THE BAND IS OPEN.   YOU WILL JUST BE CAUSING  QRM.

FURTHER  APPLICATIONS  ARE  IN  METEORSCATTER  AND

OTHER "EXOTIC" MODES OF COMMUNICATIONS.

If  you  are using this mode in  a  special  event

station  or  DX-pedition you may  type  characters

into the transmit buffer,  while the other station

is sending to you.   When he, she or even it turns

it back to you, you will transmit those characters

before the sign off sequence.

3.4.7.1   Beacon Mode Toggle/On-Off

This  function allows you to toggle  the

Beacon mode on and off.

3.4.7.2   CQ Delay Time Adjust

This  function allows you to change  the

time   that  the  program  waits  for  a

response  after  a  CQ  call  until  the

following  one  is   transmitted.    The

default   value   is  loaded  from   the

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 23

PK232COM.SYS  file.   You  may  want  to

experiment  since the actual  time  will

depend  on  the clock frequency  of  the

computer.   Make  it a small number such

as 4 or 6 when operating AMTOR or a  DX-

pedition,  and  make it a larger  number

such  as  30 when operating  on  crowded

packet channels.

3.4.8     Not used.

You  don't want to disconnect accidentally if  you

just  depress  function key 8 without the  'Shift'

key.

3.4.9     Transmit Buffer Clear

This  function is used if the PK232  continues  to

transmit  the  characters you put into  the  'type

ahead  buffer'  after  you have tried to  shut  it

down.

3.4.10    Logbook.

PK232COM  allows  automatic  logbook  entries  for

Packet  and AMTOR Beacon mode Connects,  and  semi

automatic  logbook entries for other  modes.   The

Log  file  is  compatible to that of  the  CONTEST

program  in  the CONTEST.ARC archive  and  can  be

processed  by the DBASE  Logbook Package of PC-HAM

by  G3ZCZ for indexed listings,  tracking of  DXCC

and other AWARDS,  etc.  This is ideal for special

event stations and DX-peditions, for the whole QSL

process and further statistical analyses of  their

operation has been automated.

For this function to work,  the file  PK232COM.LOG

must  be present on the disk.   If it isn't  there

it  will  be created for you.  After  one  logging

operation has occurred, edit out the initial text.

You  must  perform this editing  operation  before

appending  the contents of the logbook file to the

logbook  database of the PC-HAM  LOGBOOK  Package.

Edit  the  log  before appending it to  the  DBASE

package,  as there may be some null characters  at

the  end of the file which may be recognised as  a

blank entry by DBASE

It  is  also advisable not to let the each of  the

files get too big.

When  an  AMTOR  beacon  mode  or  packet  connect

occurs,  the connect is automatically logged.  The

connect  path is placed in the comments section of

the logbook entry.   In the non packet modes,  the

log data entry is set up for you with default  599

values for the reports exchanged.  You invoke this

function to enter the data into the log.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 24

When you invoke the function you will be presented

with  the  following  typical  menu  in  the  menu

window.

LOG ENTRY

DATE     TIME   CALL     BND  TX  RX  MODE  POWER S R   COMMENTS

87/04/27 17:28             20  599     RTTY   10   - -

Date,tIme,Call,Bnd,Tx,Rx,Mde,Pwr,qSl snt,cOmmnts or Logit (*) ?

You  should  reply with the capitalized  character

associated with each item.   For example,  if  you

want to change the Band,  enter the letter 'b', to

change the cOments,  enter the letter 'o'.   Touch

the  'ENTER'  key after you have made your  choice

and enter the text when prompted.   You will  then

be presented with an updated entry.

To  enter the data into the logbook,  use the  'l'

key, to save the data but not make a log entry use

the '*' character.   You would do that if you want

to change the band or power information.

You  may use the 'qSl sent' entry to tag the  fact

that you have written out a QSL card.

The band and power information are set up from the

default  values  stored in the configuration  file

PK232COM.SYS.   They  are displayed in the  status

window.   If  you are using the logging  function,

you  should change them when changing bands/powers

and  quit  the  logging  function  with  the   '*'

character so that you don't put a blank entry into

the logbook.

Note  that you can also use this function to enter

the odd SSB contact into the logbook file.  If you

do,  use  the  characters 'SSB' or 'FM' for  those

voice modes to remain compatible with the rest  of

the DBASE logbook package.

NOTE   Do  not use the comma character ',' in  the

comments field.   If you examine the log entry you

will see that all items are separated by  comma's.

If  you use a comma character in the comments  you

will  screw  up  the  transfer of  data  into  the

logbook  package.

The comments field in the logbook is restricted to

a maximum of 10 characters.   If you try and enter

more than 10, the surplus will be ignored.

When  you  examine the DBASE logbook file,  if  it

looks funny or the data are in the  wrong,  place,

then  you probably have the wrong number of  comma

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 25

characters in some line.  Delete the .DBF file and

re-edit the .LOG file, to make sure that there are

the correct number of comma characters.   You will

recognise the pattern readily enough.

3.5  Shift/Baudrate Control keys

The  following  keys are used to control the Shift (Wide  or

narrow)  and the baudrate in the BAUDOT,  ASCII  and  PACKET

communications modes.

3.5.1     RIGHT ARROW speeds up the baud rate.

3.5.2     LEFT ARROW slows down the baud rate.

3.5.3     UP  ARROW toggles the shift from Wide to

Narrow and back.

3.5.4     DELETE toggles the USOS function.

In  the PACKET mode the baudrate may be changed between 300,

600, 1200 and 2400 bauds.  In the other modes all baud rates

supported by the PK232 may be selected.

The  state  of each of the above functions are displayed  in

the  status  window at the top of the screen.   If  USOS  is

active,  'USOS' will be shown.  The shift is shown as 'W' or

'N'  for  wide or narrow respectively.

3.6  Additional keys

3.6.1     The HOME Key

The home key will clear the window screen that the

cursor  happens to be in at the time that the  key

was depressed,  and will then position the  cursor

at the top left hand corner of that window.

3.6.2    The DOWN ARROW key

The  DOWN  ARROW  key toggles the  Normal  Reverse

Sideband  function  in  all  communications  modes

except  MORSE.   The sideband is depicted  in  the

status  window by the up and down arrows.   If the

down  arrow  is showing,  you are  set  for  Lower

Sideband (LSB) which is the normally used one.

If you have reversed the sidebands the up arrow be

be shown in a flashing state.

If you set your transceiver to LSB you will be set

correctly for all modes.  For CW operation you may

set  your  rig to the CW position and  ignore  the

sideband setting.

3.6.3     The Alt D key (display inhibit).

This key toggles the 'Display Inhibit' mode.   Use

it  to  inhibit direct writing to the screen  when

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 26

running  under  a multi-tasking  operating  system

such as Doubledos.   It has been tested and  works

under Doubledos Version 4.0.

3.6.4     The Alt P key (printer toggle).

This key toggles the printer on and off.  When the

printer  is turned on,  any incoming data will  be

printed  in  real time.   If you intend  to  print

files  received  at  1200  bauds  in  the   packet

communications   mode,  use a print spooler or you

may  loose  data.   The printer  is  automatically

turned  off in the packet mode when  a  disconnect

occurs.

3.6.5     The Alt F key (Digipeat monitor toggle)

This  key toggles the digipeat display function on

and  off.   If  the Digipeat Display  Function  is

active, any signals digipeated through you will be

stored to the PK232COM.RUN file and a 'beep'  will

be sounded for each packet being digipeated.   The

status display window will also show a [D] at that

time.   The  MDIGI  function is set 'ON' when  the

Digipeat display function is 'ON', this means that

digipeats  will show up while you are connected to

someone.   The default state for the Digipeat Dis-

play Function is OFF.

3.6.6     The Alt R key (Message Relay toggle)

This  key  toggles the Store and  forward  Message

Relay  Capability of your system on and  off.   If

the  Message Relay Capability is  active  (toggled

on),  any  one who connects can store a message in

your  system  using the  ':QSP:  <callsign>'  NC/L

function.  If the Message Relay Capability is  not

active  (IE.  toggled off) anyone who attempts  to

':QSP: a message' will receive a ':QNO: 1' "error"

message. The status display window will show a [R]

when the Message Relay Capability is active.   The

default  state for the Message Relay Capability is

loaded from the PK232COM.SYS file.

When  the Store and forward capability is  active,

the suffix 'R' will be added to the

'*** PK232COM 1.40'

message  received  by  anyone connecting  to  your

system   to  notify  them  that  they  can   store

messages.   In this manner they don't have to  try

and get ':QNO:  1' error messages when the feature

is toggled off.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 27

4.0  PK232COM disk files

The following disk files are associated with the program.

4.1  PK232COM.COM

This is the program.  You activate it from DOS by typing the

word  PK232COM  followed by ENTER in the  normal  manner  of

invoking DOS commands.

4.2  PK232COM.RUN

This  is  the  file containing the captured  incoming  text.

This  file contains incoming data from the  PK-232  recorded

while the 'capture to disk' function was active.

4.3  PK232COM.SYS

This  file  contains the configuration information for  both

the program and the PK-232.  YOU MUST EDIT IT before you run

the  program  on  the air the first  time.   Use  your  word

processor in the NON-Document mode.    You only need to  run

this  function when you first power up the PK232 if you have

installed the batteries or if you have run YAPP or a similar

program since the last time you ran PK232COM.

The contents of the file are as follows.

4.3.1     The first line.

The first line  contains your callsign.

Please  do  not use G8BTB.   Replace  G8BTB   with

yours.   You  may  use / characters if  you  wish.

This  is  the call sign that will be used  in  the

non- Packet  modes.   You  may thus use  "portable

designators"   such  as   G3ZCZ/A,   G3ZCZ/W3   or

G3ZCZ/4X,  which you cannot do in Packet.   IF YOU

DON'T CHANGE THE CALL SIGN,  YOU WILL GET AN ERROR

MESSAGE AND THE PROGRAM WILL NOT RUN.

This is the call that will be monitored for in the

Digipeat monitoring mode.   In this mode which  is

toggled  on and off by the Alt F key,  if  someone

uses  you  as  a  digipeater  while  you  are  not

connected to anyone else, the event will be logged

to  disk.   This  mode  will  only  work  if  your

callsign  as  stored  in  this  line  is  a  valid

(uppercase only) packet callsign.

For  example,  if the callsign is G3ZCZ  then  the

sequence  >G3ZCZ> will be recognised as a digipeat

and   the   event  logged.    Thus   for   example

4X6AA*>G3ZCZ>4Z4ZB  would be logged as a  digipeat

event.

Now  if the callsign in this line is G3ZCZ/4X then

the  event  will not be recognised as  a  digireat

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 28

because  the  computer  would be  checking  for  a

>G3ZCZ/4X>    sequence   or   in   this    example

4X6AA*>G3ZCZ/4X>4Z4ZB  which it would  never  see.

This  option is inhibited for reciprocal  callsign

operators due to the AX2.5 protocol itself.

If  you have a / character in your  callsign,  and

use  the  base  call for  packet  identifying  the

location  in the CQ text,  then the function  will

work  if  you place the "portable location"  as  a

suffix.   For example, if the call is G3ZCZ/4X and

the  packet  identification  is  G3ZCZ,  then  the

digipeat  monitor  will  work.  If  the  call  was

entered  as  4X/G3ZCZ the digipeat  monitor  would

check  for the sequence >4X> and would not  appear

to work.

Store  your callsign using uppercase characters if

you want this option to work (IE.  G3ZCZ).  If you

use lower case (ie. g3zcz), the option will not be

enabled as recognition will never take place.  The

PK232 automatically converts case to upper case in

the non packet communications modes.

4.3.2     The second line.

The   second   line  contains  the  AMTOR   SELCAL

configured  for your callsign.   You use it  again

later to tell the PK232 what the SELCAL is.   This

line just tells it to PK232COM.

4.3.3     The third line.

The   third  line  contains  the  line   of   text

transmitted  by  the PK-232 in a CQ call  (in  any

mode except for MORSE). Think of this message as a

CQTEXT line.  Edit this line as well.

4.3.4     The fourth and fifth lines

The band and power information are set up from the

default  values  stored in the configuration  file

PK232COM.SYS.   They  are displayed in the  status

window.   If  you operate on another band or  your

transmitter  uses  a different power level  change

the  default  values in the file to  reflect  your

custom default state.

The  fourth  line contains the  band  information.

The  default  value is 2 for 2  Meters.   Use  the

Meter  value  for  the band to  keep  the  display

correct.

The   fifth  line  contains  the  value  for   the

transmitter power.  The default value is 10 for 10

watts.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 29

4.3.5     The sixth line

The  sixth line contains a number which tells  the

program how long to wait before transmitting a  CQ

call when in the beacon (automatic) mode.  A value

of  4 corresponds to about 2 minutes.   This value

of time will vary depending on the clock speed  of

your  computer.   You may have to experiment  with

different  values.   The default value is 0  which

inhibits the beacon mode.

4.3.6     The seventh Line

The  seventh  line configures the  LAN  store  and

forward  capacity of the system also known as  the

Message Relay Capability.  If it is a '0' (default

state)  your computer will not accept messages for

forwarding.   If it is a '1', it will.  See Appen-

dix 2 for details about LANs.

Remember,  if you configure your computer to store

and forward messages,  you have a moral obligation

to deliver them.

The  status of this function shows up as an [R] in

your  status  display and in the  connect  message

transmitted to the other station at connect  time,

as an 'R' suffix to the message when active.

4.3.7     The eighth line

This  line  contains  the  default  Communications

port.   The default value is 1.  You must leave it

set  to 1.   THIS VERSION OF PK232COM CAN ONLY  BE

USED WITH COMM1.

4.3.8     The ninth line (TNC type)

This  line  tells  the program which TNC  you  are

using.   The  PK232 command set is  slightly  dif-

ferent to those of TNC2 in the packet mode.   Thus

the  command dialog has to be changed somewhat  to

make the 'SOLO,  TRAFFIC, CQ/BEACON and MAIL' ter-

minal  modes  work correctly for different  TNC's.

NOTE  that program does not check the TNC  in  the

Non Packet modes, so for example if you try to set

a  TNC2  to AMTOR,  PK232COM will send  the  PK232

commands  to the TNC2 which should then return  an

error message.

Valid TNC's are PK232

TNC2 (MFJ 1274).

4.3.9     The remaining lines

The  remaining lines contain commands to  the  PK-

232.  For  further information about them refer to

the TNC manual.   The example in the  PK232COM.SYS

file is as follows.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 30

BEACON EVERY 0

CBELL    off

CONSTAMP ON

CHCALL ON

CHDOUBLE ON

CMSG ON

CTEXT SHALOM you have connected with G3ZCZ/4X

daystamp on

eas on

echo off

flow off

mycall G3ZCZ

MYALIAS G8BTB

MYSEL GZCZ

newmode on

NUCR ON

nulf on

UNPROTO  CQ

users 1

usos on

If  you  use a different TNC (other than the PK232) you  may

have to edit this command list.

4.4  PK232COM.LOG

This  file  contains the log entries.   It will  be  created

automatically to intialize the logging function.  The format

of the logbook file is the same as that created by the G3ZCZ

CONTEST.BAS  program  and  both files  are  designed  to  be

appended  into the G3ZCZ LOGBOOK DATABASE package  presented

as part of the PC-HAM software package.  It should be edited

with a word processor in the non document mode to delete the

first  line  and clean it up before the file is appended  to

the DBASE logging package data files.

4.5  PK232COM.DIR

This is a directory file.  You use it to store connect paths

or name/callsign relationships.  Examples are

Joe  G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR   4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

You  create  this  file with your wordprocessor in  its  non

document mode.   You must leave AT LEAST one space character

between the key word and the connect path.  When you type in

the key word, the program does not care about upper or lower

case.   If the program does not find the key word,  it  will

try to connect with whatever you typed in.

If  you  already  have  a YAPP.DIR file just  rename  it  to

PK232COM.DIR.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 31

4.6  CONFIG.SYS

You  must have a CONFIG.SYS file on the boot disk if you are

going  to  use the disk logging features.   This  file  must

contain the line

FILES = 20

See   the  DOS  manual  for  more  information  about   the

CONFIG.SYS file.

4.7  *.MBX

These  are the files associated with the stations  you  have

designated  as  store and forward mailboxes.   Thus  if  for

example  you  want  to  use 4Z4ZB as  a  store  and  forward

mailbox,  you  will create a file called 4Z4ZB.MBX using the

non-document mode of your word processor.   In this file you

will put a list of the call signs of the stations 4Z4ZB  can

connect  to reliably.   If for example you want 4Z4ZB to  be

able  to  store messages from you to  4X6AA,  4X6LR,  G3ZCZ,

ON8IK, F0WN,and G8BTB, you would put those call signs in the

file, one call on a line as follows.

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

When the QRU function is invoked either by you using Control

Function key 3 or by 4Z4ZB connecting to you and sending you

the command :QRU:  ,  any messages addressed to any of those

stations (EG.  G3ZCZ.OUT and ON8IK.OUT) will be  transferred

from  you  to  4Z4ZB just as you tried to QSP  the  messages

manually  or  G3ZCZ or ON8IK  connected to  you.   The  file

names will be changed to *.OLD.

You  must  create  one file for each  designated  store  and

forward station, ie. 4Z4ZB.MBX, G3ZCZ.MBX etc.

5.0  Initialization

5.1  Copy the files to a working disk.

5.2  Edit PK232COM.SYS. If you are upgrading from a previous

version  of PK232COM,  note that the new .SYS file  may

not compatible with the old one.

5.3  Copy the new PK232COM.SYS onto the old one.

5.4  Put  the  working disk into the active drive  (the  one

with the > prompt).

5.5  Make sure that the file CONFIG.SYS exists and  contains

the statement FILES=20.

6.0 Running the Program

6.1  Make sure that the PK-232 is connected to the computer.

6.2  Make  sure that the DOS calendar date and time are  set

to UTC.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 32

6.3  Run the program by typing the word PK232COM followed by

the ENTER key.

6.4  When  the program begins,  you will default  to  PACKET

mode on VHF (Just like the PK-232 at switch on) in  the

TRAFFIC terminal mode.  If the PK-232 is not set to the

correct  baud rate and parity type the * characters  as

explained in the PK-232 manual.

6.5  Initialize  the PK232 using the relevant  function  key

(ALT F2).

If  at this time or at any other time,  you see  'PK232

NOT  RESPONDING'  displayed in the status  window,  you

have a problem in the communications between the  PK232

and the computer.

6.5  At this time,  the computer will load the date and time

into  the PK-232.   You may thus power down the  PK-232

when it is not in use.   If you have not put a  battery

in  the TNC,  reconfigure the defaults using the ALT F2

key.

7.0 Mailbox features

7.1 Answering Machine.

The  program includes an AMTOR and PACKET mode    "answering

machine" facility.   You can leave messages on your disk (in

the same directory as the PK232COM.* files, if you are using

a hard disk),  for different stations. When someone connects

to you, if you left a message for him, he (or she or even it

as  the case may be) and only that station will  receive  it

automatically.

7.2  Mail Beacon (Annunciator).

To ensure that people know that you have left a message  for

them a 'MAIL for' list is loaded into your Packet Beacon and

transmitted  every 30 minutes (Refer to the BTEXT command in

the TNC manual) as  ':QTC:' followed by a list of calls.  If

no mail is pending then beacon transmissions are  inhibited.

This conforms to good operating practice on crowded channels

(at least inhibiting the beacon does).

The  mail beacon in the AMTOR mode is transmitted as part of

the beacon autocq message.  It is automatically updated when

a message is transmitted.   If you don't set the beacon (see

below)  you  will not transmit a mail beacon  text  in  your

AMTOR CQ message.

The  Mail  beacon text is set up either when  you  tell  the

computer to set it up using Shift function key 6 or when you

disconnect a packet QSO from another station.

In  this manner you may update the list while the program is

running,  for example by using Sidekick to write a  message,

or  preferably the NOTE function,  and then with a stroke of

Shift function key 6,  you will update the mail announcement

on your beacon.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 33

7.3  How to leave a message

You  can use the note function (Control function key  3)  or

type  the message as a document file on Wordstar,  or if you

use another word processor, you must leave the message as an

ASCII text file.

You  may also use the NOTE function built into the  program.

If you do,  the message will be given a header as if it came

in from someone else.   if you use a word processor to write

the  message,  you  will have to write in a header  or  show

somehow that the message is not 'live'.

You  just  name  the message file as  the  callsign  of  the

station  to whom you wish the message to be sent at  connect

time.  You must also give it the filetype of 'OUT'.

For  example a message for G3ZCZ would be stored on the disk

as  a  file named 'G3ZCZ.OUT',  and similarly a  message  to

4X6AA would be stored on disk as '4X6AA.OUT'.

Once the message has been sent, an entry to that effect will

be made in the PK232COM.RUN  data capture file.  The message

file  will  then  be renamed and given  the  file  extension

'.OLD'.  At that time any previously existing mail file with

the  file extension '.OLD' will be deleted.   This stops the

addressee  receiving  the message on successive connects  if

you  can't delete it for some reason (you forget)  while  at

the same time, the message is still available to you in case

you need it.

You  should use a utility program if you want the  calls  in

the  mail beacon sorted.   You may for example wish to  sort

them  alphanumerically or in the order of the date that they

were left in your answering machine.

7.4  Remote File Downloading (Packet Mode Only).

There  comes  a time when you want to leave a file  on  your

system  for someone to download later.   You can do this  by

using NC/L to control the transfer.  You must leave the file

in  a subdirectory called \PK232COM\FILES.   You must  first

create  it if it does not exist.   The station who wants the

message just asks for it using :QBM:  <filename>,  note they

do not need to use the full subdirectory path.

For example,  you have the latest ARRL DX bulletin,  and you

want to pass it on.  You could pass it to selected people by

copying  the file to individual messages with  the  filetype

'.OUT',  eg.  4X6AA.OUT,  4Z4ZB.OUT or even G3ZCZ.OUT if you

want me to get it, which wastes a lot of disk space.

On  the  other hand you could tell people that the file  was

available  for  downloading,  either in  the  CTEXT  connect

message line which everyone gets when connecting to you,  or

in individual ('.OUT') messages.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 34

If  you  include   the  following  in  your  CTEXT   message

"ARRLDX.015 now available"

people will know that to download it, they just have to type

the  line ':QBM:  ARRLDX.015' (one space  character  between

:QBM:  and the file name) to get the file.   This program is

not  designed as a BBS,  however if you copy the contents of

the \PK232COM\FILES subdirectory into a file called DIR (USE

DIR > DIR in DOS),  anyone can get a list of files the files

you have stored in it for downloading by typeing :QBM:  DIR.

Since they do not get the real directory,  just what was  in

it  at the time you copied it into the file 'DIR',  you  may

keep  "hidden"  files in this subdirectory.   You  may  thus

leave  a file for someone to download,  tell them by leaving

them a message (which they will get automatically when  they

connect)  and  no one else connecting will know that  it  is

there.

When the QRU function is invoked either by you using Control

Function key 3 or by 4Z4ZB connecting to you and sending you

the  command :QRU:,  any messages addresses to any of  those

stations  (EG.  G3ZCZ.OUT and ON8IK.OUT) will be transferred

from  you  to 4Z4ZB just as you tried to  QSP  the  messages

manually  or  G3ZCZ or ON8IK  connected to  you.   The  file

names will be changed from *.OUT to *.OLD.

You  may only use the QRU function with stations  designated

as Store and Forward mailboxes.  Put a list of stations that

you can connect to reliably in your <callsign>.MBX file  and

send  it  to other stations in your LAN also using  PK232COM

Rev  1.36 or higher.   They will then be able to dump  their

mail  messages on you and you on them.  If they do the  same

and  send  you  their file,  then you will be able  to  send

messages via them for stations that they can work.

While  QRU gives you the capability to bulk upload  messages

to  another station in your local area,  when you take  your

machine off line,  it may also be used to transfer  messages

between  two  LANs (such as the Baltimore and Washington  DC

Areas) via well sighted gateway digipeaters.

7.5 Remote Beacon shutdown.

I recognize that some stations may abuse the beacon facility

of  this  program.  I  have thus built into  the  program  a

'remote  shutdown' capability.   If someone running PK232COM

has their beacon timer set too often, connect to them in the

Packet Communications mode only at present, and tell them to

:QRT:  which  will clear their beacon mode and take them off

the air for a while at least.

APPENDIX 0. UPDATES/REVISION HISTORY

1.30 PACKET  MODE  Automatic Logging of  Connects.  Automatic  CQ

beacon added.  AMTOR Time shown in 'OVER' sequence. Callsign

sequence text added in ARQ call.  Automatic CQ beacon  added

with  logging  in  beacon  mode.    NON  PACKET  MODES  Semi

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 35

automatic logging added.   Call sequence changed to one line

except for CQ calls. Retry sequence works in all modes.

1.31 Contest  Mode.  Function key 3 operation slightly  modified.

AMTOR  answering machine added.  Overwrite of status  window

fixed.

1.32 Packet Mode Store and forward LAN :QSP: capability added.

1.33 :QBM:   message/file   downloading  added  in  Packet  Mode.

Digipeat traffic mode logging added. File handle bug.

1.34 File  handle  bug fixed,  logging/capture file  opens/closes

with  each  QSO.    Digipeat  detect  works  for  reciprocal

operators, ie >G3ZCZ>  for G3ZCZ/4X. Logbook file created if

non  existant.  Some changes in AMTOR beacon  signal  recog-

nition algorithm. PK232COM.DIR directory added.

1.35 Digipeat   detect  now  stores  data  as  well  as   address

information on disk.

1.36 LAN :QRU:  store and forward added, code condensed.  Display

of  Multiple  connect  mode added.  Changes  in  NC/L  :QBM:

changed  to :QRV:  in message mode only.   Digipeat  logging

does not indicate times of logging on/off.

1.37 AMTOR  now doesn't send Brag Tape in non beacon automatic CQ

mode.   Timeout added in PACKET Beacon mode to stop  station

connecting and holding system.

1.38 V and H (direct Packet mode VHF/HF) added to function key 6.

Code  condensed  again.  AMTOR  ROBOT override  manual  mode

added. Packet loop back call added.

1.39 CQ text change added to Alt F6.   AMTOR non beacon mode auto

CQ sends a callsign sequence and +?  when someone calls  in.

Note  feature added to Cont F3.  QRU echo's QSP  to  sending

terminal.   Printer toggle added.  RTTY 'RY' changed back to

single line in CQ mode.   AMTOR CQ works with EPROM  version

4/87.  Digipeat Display function now toggled on/off by Alt F

key (default is OFF).  CQ delay time now adjustable on-line.

Minor code changes within the program.

1.40 Thirdpary  callsign  correct in 'QNO 1' response  situation.

Alt R key (Message Relay Capability toggle) added.  TNC type

line  added to PK232COM.SYS.   TNC2 code  added.  Additional

Baud rates added.   Internal code condensed. An active Store

and  forward  Relay status is indicated in  connect  message

'*** PK232COM 1.40' by the 'R' suffix. SIGNAL mode [G] added

to  function 6.  Digipeat display function also now controls

'MDIGI'.

1.41 Packet  mode Baudrate,  shift reverses (change in 1.40)  now

done in command mode.  Delay added in Alternate function key

2 (TNC Initialise routine).  Commands transmitted in command

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 36

window, replies now show up in INwindow.

APPENDIX 1. ANOMALIES (and BUGS)

Since   the   Program  right  now  has  no  way  to   check   the

command/converse mode status of the TNC, you may see various cmd:

and  error  messages  from the TNC when using  the  function  key

commands.  Ignore them for now.  If you get an error message, try

using the function key again.

In the AMTOR Beacon CQ mode,  if the other station drops the link

while  you  are  recording the text,  the program will  time  out

before  logging  the  QSO and restarting  the  CQ  sequence.   If

someone else calls in before the timeout has occurred,  the  text

from both stations should be captured to disk,  but the log entry

may not show both stations,  thus the 'LOG' must be compared with

the 'RUN' file for QSL purposes.

In  the AMTOR Beacon CQ mode,  if another stations sends a +?  in

the  FEC mode,  the beacon will attempt to send the  PK232COM.TXT

file.  The PK232 however is not in the converse mode at that time

and will interpret the text of the file as commands.   If by some

chance,  the  text corresponds to a valid command the PK232  will

execute it and you may not notice that it did until you begin  to

wonder why nobody is replying to the beacon.  As such I recommend

that the first character of each line in the file PK232COM.TXT be

a non valid command character such as a number or a period (.) or

an other punctuation sign.

PK232COM  versions 1.00 to 1.39 were written for a PK232 with  an

Oct 1986 ROM.   There may be differences with later ROMs. Version

1.39 works with the April 1987 ROM.

APPENDIX 2. Local Area Network Capablity

VHF  Packet  radio systems can be considered as part of  a  Local

Area  Network (LAN) in which messages can be left by one  station

in the computer belonging to a second station.  At HF the same is

true,  but  the area becomes greater.   The basic problem here is

that  people  can only send and receive messages to or  from  you

when you are on-line.   To compensate for this, BBS stations were

developed  which allowed both messages and bulletins to be stored

by anyone for later retrieval.

PK232COM  allows you to store messages in your computer so as  to

use  it as an automatic answering machine.   Thus by storing  the

messages in a file called by the name of callsign,  with the file

extension  '.OUT',  anyone  connecting to the computer  will  get

their  message without having to request it.    PK232COM  Version

1.32 extends that concept to cover the eventuality of you wanting

to take your computer off-line for some reason.  You may now load

the  message  into any other computer you can  connect  to  using

elements  of  the Q code adapted to the High Level  Network  Com-

munications  Language  (NC/L)  proposed in my book  Software  for

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 37

Amateur Radio,  (Book number 1560) published by TAB  Books,  Blue

Ridge Summit, Pa. 17214 USA.

The protocol is as follows.

When  connected to someone who has their computer configured as a

host,  if  you  want to store a message you  send  the  following

instruction   to  the  other  station  :QSP:   <callsign>   where

<callsign>  is  the call of the station that the message is  for,

not  the callsign of the host station in whose computer  you  are

storing the message. [Note use only one space character after the

:QSP:  ].   PK232COM sends the :QSP:  message automatically using

Control Function Key 3.

For  example if you want to store a message for 4Z4ZB in  4X6AA's

computer  which is configured as a Store and Forward system,  you

would  first connect to 4X6AA and then send the message as

:QSP: 4Z4ZB .

The  other computer will respond either with a  statement  saying

that  it is ready for you to go ahead,  or send a message  saying

that  it can't comply.   If it is ready you get a positive  reply

which will take the form :QRV:   <callsign> which if you know the

Q code, means " I am ready to accept a message for  <callsign>".

At this time you may go ahead and send the message.   If you type

the message at the keyboard,  you may use either a control Z (^Z)

character or the character sequence :EOF:  followed by a carriage

return  (the ENTER key) to terminate the message.   If  you  have

first  written the message into a text file you may then send  it

using function key 2  and the termination sequence is automatic.

Once  you have completed the message,  the other (host)  computer

will  either reply that the message has been successfully  stored

or give you an error message.

If  the  message  is stored and ready to be sent  next  time  the

addressee  connects  to that computer,  you will see the  message

:QSL: on your screen.  If something went wrong, you will get back

a  negative message taking the form :QNO:  followed by a  number.

The number tells you why the operation failed.

Message Format

The  message is stored just as if you had left it in your  system

(except that a header is added identifying the time of  reception

and  the call of the sending station).   It will contain a header

showing  the  date and time it was received  and  your  callsign.

Should a message for that station already be in the system, yours

will be appended to it.  In the event the your upload is aborted,

the  amount  of text received before the abort occurred  will  be

stored as the message.

When  you disconnect from the host station,  its mail beacon will

be updated.

Once the message is loaded in the host, it can only be deleted by

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 38

the operator of the host station.  When the addressee connects to

the host and receives the message, the file type will change from

'.OUT' to '.OLD'.

File downloading

See  section  7.4 for the use of :QBM:  for downloading  text  or

ASCII files from another PK232COM system.

APPENDIX 3     BASIC NC/L DICTIONARY

Messages  can  be  transferred into any other  computer  you  can

connect to using elements of the Q code adapted to the High Level

Network  Communications  Language  (NC/L)  proposed  in  my  book

Software for Amateur Radio,  (Book number 1560) published by  TAB

Books, Blue Ridge Summit, Pa. 17214 USA.

To  receive  a message,  do nothing,  you will receive your  mail

automatically when connecting/linking to a PK232COM station.  You

cannot read messages adressed to another person.

:QSP:     To leave a message, send

:QSP: callsign

the callsign is that of the station you wish to leave a

message for.

:QRU:     To  upload messages from one PK232COM (1.38+) system to

your computer send

:QRU:

:QBM:     To download a message, send

:QBM: filename.type

The filename.type is the file you want. For example

:QBM: dir

:QBM: help

:QBM: pk232com.doc

:QRT:     To shut down a packet mode beacon station who is causing

QRM, connect to the station and send

:QRT:

Computer responses

:QSL: callsign

Confirms receipt of message to that callsign.

:QRV: callsign

The computer is ready you to send the message.  End the

message  with  a  control  Z  (^Z)  character,  or  the

sequence :EOF: .

:QNO: 'NO' or error

QNO Error Values.

The following error numbers are associated with message store and

forward operations.

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 39

1    Computer not configured as Store and Forward system.

2    Requested ASCII file/ message (:QBM:) does not exist.

3    You  made an error in the name of the callsign for whom  the

message is intended (It must be at least 3 characters long).

4    File creation error in host system.

5    Error  occurred  during reception and  storage  of  message.

Could be that the computer ran out of space on the disk,  or

something else went wrong in storing the message.

6    :QRU: You are not authorised as a store and forward mailbox.

7    :QRU: Error in opening <callsign>.MBX file.

8    :QRU: Error in closing <callsign>.MBX file.

9    :QRU: Sequence  Error in callsign of message to go.  The bad

callsign will be shown after the error number.

:QTC:  messsage list.

This is not quite a response, but it precedes a list of callsigns

for  whom messages are stored up on a computer.   It is  used  in

Packet Beacon transmissions or AMTOR Beacon mode CQ calls.

PROPOSED EXTENSIONS

:QYU:     YAPP format file upload.

:QYD:     YAPP format file download.

APPENDIX 4. USING OTHER TNC'S.

PK232COM  is  designed for the PK232.   However the  PACKET  mode

capabilities  can  be  used  on other  TNC's.   There  are  minor

differences in the command dialog between different  TNC's.   The

dialog  differences affecting the operation of PK232COM are built

into  the  program  and set when the TNC type is  read  from  the

PK232COM.SYS file at the program load time.  The non packet modes

will obviously result in error messages if attempted on a  packet

mode TNC.

The   following  notes  apply  to  different  TNC's  or   digital

communications controllers.

4.1 TNC2 (MFJ 1274)

4.1.1     Default Commands.

Edit  the  PK232COM.SYS file taking out the  PK232  specific

commands,  namely  anything  that is not mentioned  in  your

manual.   Add  in the default state for the TNC commands not

included  in the file and make sure that you set  AUTOLF  ON

and MCOM ON

4.1.2     Program differences.

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

MAIL Packet terminal mode (Alternate Function key 4).

PK232COM.DOC (c) G3ZCZ 1987

PK232COM VERSION 1.41  PAGE 40

CQ/BEACON Packet terminal mode (Alternate Function key 4).

TNC Transmit Buffer flush (Control Function key 9).

Digipeat display and logging function (Alternate F key).

APPENDIX 5. NOTES

I'm   always  interested  in  your  bug  reports,   comments  and

suggestions for changes.

PK232COM.DOC (c) G3ZCZ 1987

#########################################