WEFAX     Weather Satellite Facsimile     by CHARLES JACKSON     Antic Program Editor     Tune in a weather satellite and downlink a few picturesfrom space!  A standard shortwave radio plus this issue'sWEFAX Decoder software and WEFAX Interface board are all thatyour Atari needs for receiving and displaying WeatherFacsimile pictures from the satellites of the US and othernations, as well as facsimile photos from newswire services.     This Weather Facsimile system is the most ambitiousproject ever published by Antic.  It required more than sixmonths of intensive research, programming and hardwaretinkering.  But the result is the most versatile andfriendliest weather satellite software available for theAtari 8-bit and ST computers.     The reason this project turned out to be so demanding isthat its success depends on many factors -- includingsoftware, hardware and the forces of nature Especiallyimportant are what type of radio and antenna you use, and thearea you live in.     You need a shortwave radio capable of receiving SSB(single side-band) signals.  Inexpensive "all-band" portableswith telescoping antennas will not suit our purposes.  Wedeveloped these programs using a Radio Shack DX-302 attachedto a 25-foot longwire antenna.  But for good results, youshould use the best receiver and outdoor shortwave antennayou can afford.     Also, even if you type in the program perfectly andbuild the interface circuit without a hitch, you could be outof luck if you live in an area with poor shortwave reception.WEFAX signals are clearest in suburban and rural areas.  Ifyou live in the center of a large city, you may have troublereceiving a clean WEFAX picture.     PROJECT ELEMENTS     Before you can use the Decoder programs, you'll have tobuild the WEFAX Interface described in this issue.  Theinterface is a simple circuit that can be built for under$20.     WEFAX.EXE is for 8-bit machines, WEFAX.TOS is for theST. Copy WEFAX.EXE to anotherdisk and rename it AUTORUN.SYS, then follow the instructionsin the accompanying articles.     Listings 2 and 3, FAX.M65 and FAXA.M65, contain theMAC/65 source code for WEFAX.EXE.  You do not need to type inthese listings to use the WEFAX program.     Listing 4, WESIM.BAS, is a BASIC program that simulatesa WEFAX signal.  If you've never heard a WEFAX signal, thisprogram will create one for you.  Listing 5, WETST.BAS, is aBASIC program that helps you test your WEFAX Interfacecircuit.     The ST version of WEFAX Decoder is WEFAX.TOS and you'llfind the article explaining it in this issue's ST Resourcesection.     THANK YOU!     Finally, Antic would like to thank all the people whohelped us with this project, including:  Dr.  Ralph Taggart,Dr.  Marty Goodman, CompuServe Vice-President Sandy Trevor,Vic Moore, Jim Grubs, Chris Elmquist and Wayne Day.  Specialthanks to the staff of the San Francisco Area Office of theNational Oceanic and Atmospheric Administration, includingWilliam Pettyplace and Jan Null.     SUGGESTED READING     If you want to learn more about Weather Satellitetransmissions, here are some of the best sources to get youstarted:     The New Weather Satellite Handbook, by Dr.  RalphTaggart.  This is the definitive text on WEFAX.  Formerly outof print, an updated edition of this popular book isavailable from the author for $12.50.