BY MATTHEW RATCLIFF     (synopsis)     This look inside the Atari XE Game System includes twoprograms.  XEGS Manager is a utility for convenient controlof all the Game System's built-in features, but many of itsoptions can also be used on the other Atari XL/XE computers.The second BASIC program is a short "spray-painting" routinefor testing light guns.     Atari executives asked the heads of several major toystore chains which product they'd rather sell -- the powerful65XE home computer for about $80, or a fancy new game systemfor about $150.  The answer was, "You can keep the computer,give us that game machine!"     This "game machine" is what we now know as the AtariXEGS, the XL-compatible Extended memory Game System.  It'ssimply an enhanced 65XE in a game machine package.  It's alsoa brilliant idea.  The XEGS has been selling out almost asfast as toy stores can get them in.     The XEGS may not seem like such a hot idea to seriousAtari computer users.  But just think about it.  If you wereafraid of computers or don't have the foggiest idea what todo with one, you'd have absolutely no interest in an Atari65XE -- even if it could play great games.  However, you'dprobably have no compunction about buying a great video gamesystem, the XEGS, as a new addition to the familyentertainment center.     Now we'll take a close technical look at the Atari XEGS.I'll explain how some of its changes in physical design haveaffected the operating system software.  I'll also presentthe XEGS Manager, a utility for controlling all the built-infeatures of the XEGS.    KEYBOARD     The keyboard of the XEGS is detachable.  When notconnected, the XEGS console looks (and acts) like just a tamelittle game console.  In fact, Missile Command is turned onautomatically.     Plug in the keyboard and turn on the machine without anexternal cartridge, and you're running Atari BASIC, RevisionC.  The keyboard is virtually identical to the one found onthe 130XE.  It's mushy, but you can get used to it.     The cable on the XEGS keyboard is quite short.  Thereare two brackets at the top of the keyboard case which lockneatly under the front of the console.  The keyboardconnector is a standard DB15 female.  My first XEGS projectwas to construct an extension cable for the keyboard.  I findit far more comfortable to type for a long time with thekeyboard on my lap than on a desk.     Inside the XEGS keyboard case is a small circuit board.On it you will find some resistors, capacitors and two CD4051chips, which decode the keypresses and send an internalkeycode back to the POKEY chip in the XEGS.  I already madean adapter cable to connect the XEGS keyboard to the 800XLjoystick ports.     So far my efforts at "scanning" the external keyboardmanually have failed -- the POKEY chip does thisautomatically in the XEGS -- but I hope to bring you a laptopkeyboard utility program that lets you use the XEGS keyboardon any other Atari computer with minimal hardware hacking --just a cable.    MEMORY     There are only two RAM chips in the XEGS, which delivera full 64K of RAM.  They're Texas Instruments TMS4464-1264K-by-4-bit chips.  In the XL/XE computers, eight64K-by-1-bit chips are used.  Fewer chips improvesreliability and generally reduces the cost of producing themachine.  In fact, the XEGS contains a total of only 17chips.     ICD has indicated that it is working on a RAM upgradekit for the XEGS similar to the RAMBO XL for the Atari 800XL.I've already received a 128K RAM upgrade kit from InnovativeConcepts for my XEGS, making it fully 130XE-compatible.  I'llreview this in a future issue.     Using higher-density ROM chips, the updated XEGSoperating system had 8K of spare ROM.  Atari decided to usethat extra ROM for the Missile Command game, which isbank-switched in and out much like how Atari BASIC is toggledon and off.  Missile Command can be enabled by holding the[SELECT] key during power-up when a keyboard is connected.When in BASIC, you can enter BYE to get to the Self Test.From there, pressing [RESET] while holding the [SELECT] sendscontrol to Missile Command.  Press [RESET] in MissileCommand, while holding [OPTION], to return to the Self Test.Press [RESET] by itself to return to BASIC.  (Reboot if adisk drive is connected.)    LIGHT GUN     The XEGS comes with a light gun called the XG-1.  TheNintendo light gun is more accurate -- if something is linedup in its sights, that's exactly what you hit.  Not so withthe XG-1.  You'll find that it often shoots to the left orright, depending on the software you're running.     The XG-1 is simply a specialized light pen.  Light pensupport was built into the earliest Atari computers, but itnever really caught on.  In the shape of a gun, the light penhas brought a whole new dimension to video gamesapplications.  The light pen horizontal position, LPENH, canbe PEEKed at memory location 564, and the vertical position,LPENV, is found at location 565.     Light gun values range from 0 to 227.  You will noticethat your horizontal readings are quite odd.  Try the sampleprogram below, and notice how the GUN-X readings vary as yousweep the gun across the screen, left to right.     10 LPENH=564     20 LPENV=565     30 GRAPHICS 0:POKE 752,1:POKE 712,15     40 POSITION 0,0:FOR I=1 TO 4:?  "0123456789";:NEXT I     50 ?  "GUN-X=";PEEK(LPENH);"  "     60 ?  "GUN-Y=";PEEK(LPENV);"  "     70 ?  "TRIGR=";STICK(0)     80 GOTO 50     Point the gun to the far left of the display and GUN-Xwill read about 88.  Moving from left to right, the readingwill reach 227 at about column 34.  Then suddenly it drops to0 and increases again to about 30 at column 39.  This offsetis due to the delay between when a pixel is actually lit onthe display and when the information is relayed from thelight gun sensor to the POKEY chip, which latches an internalscan counter for the pen reading.     The old Atari 400/800 Hardware Technical Referencerecommends a "calibration procedure" each time the light penis used, so that the software can compensate for this offset.A calibration procedure would improve the accuracy of thelight gun.  But Atari's Bug Hunt and Barnyard Blaster gamesboth have "hard-coded" values -- different ones in fact.While Bug Hunt appears to shoot slightly to the left,Barnyard Blaster seems to shoot a tad to the right.  The Yreadings for the gun are more predictable, equal to half thenumber of the currently displayed scan line.  You'll noticewith your test program that GUN-Y only varies from about 17to 115.  Note that you get much better performance out of thelight gun near the screen edges, when you use a light coloredborder achieved with the POKE 712,15 above.     You'll need to perform some computations to adjust forthese unusual readings, to convert gun coordinates to screencoordinates.  Different conversion factors are required foreach graphics mode (and P/M graphics).     The gun won't return reliable readings at all if theintensity of the display is too low.  That's why the screensfor Atari light gun games may be brighter than usual.  Thegame screen will momentarily flash white whenever you pressthe trigger in either Bug Hunt or Barnyard Blaster.  Whilethe screen is all white, the software reads the gun positionand provides the most accurate values.     Listing 1 presents a simple Graphics mode 8"spray-painting" program for testing the XG-1.  Dots aredrawn whenever the fire button is pressed.  Try holding thegun very steady to see how much jitter you get in thereadings.  These inaccuracies are reflected in your games aswell.  I feel that, at the higher levels of play, Bug Huntand Barnyard Blaster both require more accuracy for continuedplay than the XG-1 can deliver.     Atari has informed me, however, that the XG-1 and arevised Bug Hunt will be released as a separate package.  Ifyou're tired of waiting, you may wish to pick up the Segalight gun, for about $25 (when on sale) and modify it for theAtari.     To modify the Sega gun for the Atari, you'll have to cutoff the incompatible connector.  The wires must be strippedback and soldered into an Atari joystick connector asfollows:     SEGA GUN     ATARI JOYSTICK PORT     Blue wire    Pin 1  stick FWD     Gray wire    Pin 6  trigger     Green wire   Pin 7  +5 volts     Black wire   Pin 8  Ground     Because of the close-fitting connections for the XEGSports, don't wire in a DB9 female connector that has "ears."Most joysticks don't have wires for unused signals, socutting up an old joystick cable may not work.  Specifically,an Atari joystick does not need the +5 volts, so there isn'tlikely to be a wire connected to Pin 7.  However, you canfind joystick extension cables at Radio Shack, which have allnine pins wired from male to female.  ANTIC disclaimsresponsibility for any damages that might occur duringimproper implementaton of this, or any, hardware modificationproject we publish.     Once it's all hooked up, you'll notice that gun fireswhen you release the trigger, which is annoying.  The Segatrigger wiring is the opposite of what the Atari light gunuses.  To rewire the trigger switch, remove the five screws(one is under the Sega logo on the side).  Find the triggermicro switch with three connections.  Wire to the normallyclosed contacts instead of normally open.    XEGS MANAGER     Listing 2, XEGSMGR.BAS, is a BASIC loader that lets youcreate a machine language file on disk.  Type it in, check itwith TYPO II and SAVE a copy before you RUN it.  The fileXEGSMGR.EXE can then be loaded from DOS or renamedAUTORUN.SYS.  Many of the program's features also can beapplied to Atari XL/XE computers.     Normally you must press the [OPTION] key at boot time todisable BASIC and go directly to

DOS.  And once BASIC is off,the only way to get it back on is rebooting.  Option 1 ofXEGS Manager is to turn internal BASIC on, and option 2 turnsit off.     Disabling BASIC while in DOS provides an additional 8Kbuffer for file copying.  This is an important feature forowners of a single drive.  Quite often, BASIC must be offbefore certain machine language files can be loaded and run.The XEGS Manager eliminates the need of rebooting every timethat BASIC must be re-enabled.    SELF TEST     The XEGS Self Test lets you test the computer's soundregisters, keyboard, and memory.  However, BASIC is notturned off automatically when Self Test is run from the BYEcommand.  This means that the 8K of RAM under BASIC isn'ttested.  Option 3 from the XEGS Manager lets you run the SelfTest with BASIC off, so that the maximum ammount of RAM istested.    RAM OS     All of the operating system of the XEGS (and 64K or moreXL/XE machines) is "shadowed" by RAM.  Some disk operatingsystems, such as DOS XL and SpartaDOS, use hidden RAM formany of their own functions.  However, if you're using AtariDOS 2.0 or 2.5, then there is a lot of RAM going to waste inyour machine.     Option 4 of the XEGS Manager lets you enable a RAm-basedoperating system so you can do some real "hacking" --disassembling and adjusting parts of the XEGS operatingsystem to suit your needs.  Even if you're not a hacker,there are other practical features of a RAM OS.     Once the RAM operating system is enabled, you areprompted for a disk drive number, 1-8 or 0 to exit.  A customfont can be loaded in place of the standard one in the OSROMs:  enter the drive number to display a directory of all.FNT files.  Then enter the name of the font file to load, orsimply press [RETURN] to change drives or disks.     You needn't enter the drive specifier or extender -- theXEGS Manager will take care of that for you.  The font fileis loaded into memory at $E000 (57344).  Then you're promptedto (1) repeat the process and try a different font, or (2)exit.  (You will find some font files on this month's disk asa bonus.)     Your RAM OS and font are reset-proof, too.  Pressing the[RESET] key causes the XEGS to re-enable the ROM-basedoperating system, but a special handler in Page 6 of memorytakes control after that.  The handler converts the ROM backto RAM, recopying all the essential parts of the ROM OS, incase part of this RAM area got clobbered while you werehacking about.  The handler does not recopy the ROM OS font,however, leaving yours intact.  Should your RAM font getgarbled somehow, press [RESET] while holding the [START]console key to return to the ROM font.  Each time that[RESET] is pressed, a RAM OS prompt is displayed at the topof the screen as a reminder.     If you enable a RAM OS while running SpartaDOS, theXEGS Manager detects it and prevents the installation andsubsequent system crash.  The Manager does not automaticallydetect any other DOS, such as DOS XL, which may crash when aRAM OS is enabled.    MISSILE COMMAND     Again, if you turn on your XEGS without the keyboardconnected (or hold down the [SELECT] key at power-up),Missile Command fires up automatically.  Option 6 of the XEGSManager will get you into Missile Command without having tomess with any console keys or power switches.    CONCLUSIONS     The XEGS is a superb little computer.  It's still ahacker's system too.  I've found that the PBI ROM routinesare intact, which means that you should be able to hack inyour own custom PBI connector and use the XEGS with ICD's MIOboard, if you're a real solder jockey.     The XEGS has brought along a lot of new software too,something Atari was counting on.  Much of it includesrepackaged classics or cartridge conversions from disk-basedsoftware, but there are a few new titles such as Battle Zone.Atari's new 256K bank-switch cartridges are not likely to bepirated.  This means that the piracy threat for 8-bit Atarisoftware should be minimal, thus attracting more new softwarevendors from the traditional Apple and Commodore markets.     If Atari can provide a responsive cartridge productionservice for third party software vendors (something the oldAtari never would have done), then we're likely to see thesoftware base for the 8-bit Ataris grow with the popularityof the game industry, which is definitely on the rise again.    (bio)     Matt Ratcliff is a St. Louis aerospace engineer and alongtime ANTIC contributing writer.#############################################################################################################